1.0 OBJECTIVE

To provide a standardized procedure for engraving identification numbers or codes on laboratory glassware to ensure traceability and prevent cross-contamination or misuse.

2.0 SCOPE

This SOP applies to engraving of glassware such as pipettes, burettes, standard flasks, and measuring cylinders.

3.0 RESPONSIBILITIES/ACCOUNTABILITIES

- QC Officer/Lab Technician: Responsible for engraving and ensuring compliance with this SOP.

- QC Head: Responsible for assigning unique IDs and approving engraving layout.

- Quality Assurance Manager: Ensure the procedure is follows.

4.0 DEFINITIONS AND ABBREVIATIONS

Engraving: A permanent marking method using an engraving pen/tool to etch identification marks on the glass surface.

Glassware: Laboratory apparatus made of glass, used for precise measurements and chemical handling.

5.0 PROCEDURE

Preparation

- Retrieve the clean and dry glassware from the storage area.

- Clean the area to be engraved with alcohol wipes and allow it to dry.

- Review and assign a unique identification code from the glassware engraving logbook as follows.

Glassware engraving logbook LGB/QDG/CL/012/001

| S/N | Glassware name | code | Model/Type | Date issue | QCH sign/date |
| --- | --- | --- | --- | --- | --- |

e.g for Pipette, the first one for such laboratory will have the engravement as FPP01

where:

F – Infusion

PP – Pipette

01 – serial number of the glassware in such laboratory

See guide below for others which is not limited to these.

| Glassware | Code | Code (Infusion) | Code (Syrup) |
| --- | --- | --- | --- |
| Pipette | PP | FPP01 | SPP01 |
| Burette | BR | FBR01 | SBR01 |
| Measuring cylinder | MC | FMC01 | SMC01 |
| Conical flask | CF | FCF01 | SCF01 |
| Standard flask | SF | FSF01 | SSF01 |
| Desiccator | DS | FDS01 | SDS01 |

Engraving Process

- Using a permanent marker, write the ID code on the area to be engraved.

- Recommended positions is as follows:

- Pipette: Near the top (bulb or mouthpiece end)

- Burette: Near the top, just below the clamp area

- Standard Flask: On the neck or shoulder area

- Measuring Cylinder: Near the top or base

- Hold the glassware firmly or secure it on a stable surface.

- Using the engraving pen, slowly and steadily trace the marked code.

- Repeat the tracing once or twice to deepen the engraving for permanence.

- Wipe off the remaining marker with alcohol wipes.

- Inspect the engraving for clarity and completeness. If unclear, re-engrave as necessary.

Post-Engraving

- Clean the glassware again with alcohol and inspect for cracks or damage.

- Record the engraved code, date, and type of glassware in the logbook.

- Return the engraved glassware to the storage area or issue to the respective lab unit.

Acceptance Criteria

- Engraving must be legible, permanent, and placed at the specified location.

- Glassware must remain free from cracks, chips, or weaknesses post-engraving.

Materials And Equipment

- Diamond-tipped glass engraving pen or electric engraving tool

- Protective gloves

- Safety goggles

- Clean cloth

- Permanent marker (for initial layout)

- Alcohol wipes or 70% IPA

Safety Precautions

- Always wear gloves and goggles during engraving.

- Perform engraving in a well-ventilated, clean area.

- Avoid inhaling glass dust and ensure glassware is empty and clean before engraving.

- REFERENCE/ENCLOSURE

- GLP (Good Laboratory Practice) Guidelines

- ISO 17025: General requirements for the competence of testing and calibration laboratories

- TRAINING INVOLVEMENT

| Quality Control Head | Chemical Analyst |
| --- | --- |
| Doc. Unit | IPQA |

8.0 DISTRIBUTION AND DOCUMENTATION CONTROL:

| MASTER COPIES | QUALITY ASSURANCE |
| --- | --- |
| CONTROLLED COPIES | QUALITY CONTROL LABORATORY |

9.0 REVISION HISTORY

| CURRENT REVISION | SUPERSEDE | REASON FOR REVIEW |
| --- | --- | --- |
| ### 00 | ### Nil | First issue |