- OBJECTIVE:

To provide written instruction on microbial limit testing on finished product

- SCOPE:

To provide written guidelines for carrying out microbial limit tests on finished products.

- RESPONSIBILITY/ACCOUNTABILITY:

- The microbiologist is responsible for following the procedures outlined in this STP.

- The Quality control manager is to ensure the procedures outlined are followed.

4.0 ABBREVIATIONS AND DEFINITIONS

- TAMC: Total aerobic microbial count

- TYMC: Total combine yeast mould count.

- EEB: Enterobacteriaceae enrichment broth.

- CFU: Colony forming unit.

- TNTC: Too Numerous To Count

- PROCEDURE

Method used

- Pour-plate method: for isolating and counting the viable microorganisms present in a liquid sample, which is added along with or before molten agar medium prior to its solidification.

Requirement for Pour Plate Method

- Liquid Samples (or suspension of the solid sample)

For the pour plate, the sample must be either liquid or in suspension form. The solid sample must be suspended in a suitable solvent that doesn’t influence the growth of any microorganisms or react with any material of the media.

- Suitable Solid Culture Media

Specific culture media is used for the isolation and differentiation of suspected (or specific) bacteria. The culture medium is a solid agar medium that is melted and is at a temperature of 45°C.

- Petri Plates

sØ90mm (OD) Petri plates are used. They must be sterile and must be labeled before pouring samples or media.

- Test-tubes

Test tubes are needed for serial dilution during sample preparation. The tubes must be sterile.

- Sterile Distilled Water (or Sterile Broth)

Either distilled water or broth can be used for serial dilution. They are also used for dissolving the solid or semi-solid sample.

- Micropipette (or Graduated Pipette)

A micropipette of 0.1 mL or 1 mL capacity is required for measuring the sample during serial dilution and sample inoculation.

- Other Microbiological Laboratory equipment such as Lamina Airflow, Incubators, ovens etc. must be verified in-house and external calibrator.

General Procedure

The general procedure for performing the pour plate method is stated as follows:

- Arrange all the requirements, put on the PPE, sterilize the work surface, and set up the laboratory types of equipment.

Sample preparation:

- If the sample is in liquid form, Prepare a dilution of 10-1and use different dilutions.)

- If the sample is in solid or semisolid form, dissolve it in sterile distilled water or sterile broth, or any other solvent. (Generally, 1 gm sample is mixed with 9 ml of solvent to get the concentration of 10-1 gm/mL.)

Media preparation:

- Suitable media (soya bean digest agar for bacteria, and Sabouraud Dextrose Agar for fungi) are prepared and autoclaved as per SOP for media preparation.

- The media is allowed to cool to about  45°C (maximum up to 50°C), but don’t let it solidify.

Note: (If the media is prepared already and solidified, melt it by placing it over a water bath or other heat source)

Arrangement of Petri-dishes inside the inoculation room

- Arrange sterile Petri plates. Label at the edge of the bottom of the plate with the dilution factor, date, name, sample ID, and other required information.

Inoculation:

Method (Microbial Enumeration Test)

- Determination of total aerobic microbial count (TAMC) and total combined yeasts and molds count (TYMC) by the pour plate method.

- Dispense 1 ml of diluted sample in the center of the Petri plate using a sterile micropipette or calibrated pipette.

- Open the lid of the prepared media, flame its mouth.

- Pour about 15 mL of sterilized molten media at the appropriate temperature above the sample.

- Close the lid of the plate then mix the sample in the media properly by gently swirling the plate. The plate is generally swirled in an “S” or “8” shape.

- Allow the media to completely solidify.

- Incubate the plate in an inverted position under suitable conditions

- Incubate the plates of Soybean Casein Digest Agar at 30°C-35°C for 2 days for TAMC and the plates of Sabouraud Dextrose Agar at 20°C to 25°C for 3 days

TEST FOR ABSENCE OF PATHOGEN

Test for E. coli:

- Measure 10ml /10g of the samples, dilute into 90ml of sterile tryptic soy broth

- Incubate at 30-350c for 48hours

- Transfer 1ml of the dilution into 10ml of MacConkey broth and incubate at 30-35Oc for 48hours

- Subculture on MacConkey Agar or Eosin Methylene blue Agar and incubate at 30-35Oc for  48hours

- For negative control, proceeding same as above except without sample, streak a portion of incubated sterile MacConkey broth medium on the surface of sterile MacConkey agar or Eosin Methylene Blue Agar.

- Incubate plates at 30-35° for 48 hours.

Result Interpretation

- Following the incubation, developed colonies are observed. Each colony will account for one viable microbial cell or one colony-forming unit.

- If all the colonies are of the same type, then we can predict that the sample contained only one type of microbial genera. However, if there are different genera or species producing similar types of colonies, it is require to carry out identification test.

- If the colonies are of different morphology, then we can conclude that the sample contained a mixed population.

- Count the colonies and calculate CFU/mL by using the formula:

CFU/mL  =Total number of colonies obtained x dilution factor

Volume of sample used (aliquot)

- This will give the total number of viable microbial cells present in the given sample.

- For optimum count, the number of colonies must be between 20 – 200 CFU/mL. Beyond this limit, the whole procedure must be repeated.

- If the number of the colony is less than 20, it is suggested to use the sample of lower dilution, whereas, if the total number of colonies exceeds 200, it is suggested to use the sample of higher dilution on successive repeats.

- The result should be recorded in the Standard Testing Procedure form on finished product - BML/QA/STP/FPX/YYY/F03 following the Standard testing procedure SOP BML-QDG-QA-105

- If the colonies are fused or the whole plate is covered with a single colony, then report as “too numerous to count” (TNTC) and repeat the process of taking the sample at a higher dilution.

- All samples must free of pathogens

- When an acceptance criterion for microbiological quality is prescribed it is interpreted as follows:

－101CFU: maximum acceptable count＝20,

－102CFU: maximum acceptable count＝200,

－103 CFU: maximum acceptable count＝2000.

Precautions

- Follow proper safety protocols.

- Every Petri plate and media in use must be sterile.

- The analysis must be carried out under conditions designed to avoid extrinsic microbial contamination of the product to be examined. The precautions taken to avoid contamination must be such that they do not affect any micro-organisms which are to be revealed in the test.

- Ensure the inoculation room is cleaned, the work bench and laminar flow is disinfected with 70% Isopropyl alcohol

- The water or media used in serial dilution or to dissolve the solid sample must be sterile.

- The solvent used to dissolve the solid sample must be sterile and must not have any growth-supportive or inhibitory effect against any microorganism.

- The sample must be diluted enough so that the viable microbial load falls between 20 – 200 CFU/mL. Above this range, it will be difficult to count the colony and the colonies may fuse together. Below this range, the result is reported as not significant. The process must be repeated under the same condition if the colony count is below 20 CFU/mL or above 200 CFU/mL.

- Accurate measurement is compulsory during serial dilution and media preparation.

- Use micropipette or standard calibrated tube while transferring sample in the Petri plate or in media tube or bottle.

- The permitted volume of media is 15 -20 mL per Ø90mm (OD) Petri plate by the USP.

- The sample must be properly mixed with the molten agar before solidification.

- The temperature of media during pouring or mixing with the sample is very important. It must be   450C (500C is the maximum temperature and must not be more than this at any cost). The temperature below 450C is not recommended because the media will either solidify or clumps may start developing.

- Each Petri plate must be labeled prior to pouring of media and sample. There must be labeling about dilution factor.

- While dispensing the sample, the labeling must be checked and the information about the dilution factor must be matched with the concentration of the sample.

MATERIALS NEEDED

- Bucket Autoclave

- Hot air oven

- Laminar airflow

- Incubators

- Petri dishes Ø90mm (OD)

- Conical flask

- Culture media (Casein digest soya bean broth, Casein digest soya agar, sabouroud dextrose agar, MacConkey broth, Eosin Methylene Blue agar)

- Test tubes

- Cotton wool

- 70% Isopropyl alcohol

- Hand gloves

- Nose mask

- Micropipette (or Graduated Pipette)

- REFERENCE/ENCLOSURE

- Standard Testing Procedure form on finished product - BML/QA/STP/FPX/YYY/F03

- TRAINING INVOLVEMENT

| Quality Control Head | Quality control microbiologist, Laboratory assistant |
| --- | --- |

8.0 DISTRIBUTION AND DOCUMENTATION CONTROL:

| MASTER COPIES | QUALITY ASSURANCE |
| --- | --- |
| CONTROLLED COPIES | QUALITY CONTROL LABORATORIES |

9.0 REVISION HISTORY

| CURRENT REVISION | SUPERSEDE | REASON FOR REVIEW |
| --- | --- | --- |
| ### 00 | ### Nil | First issue |
| ### 02 | ### 01 | To Update |
| ### 03 | ### 02 | - Inclusion of enclosures format - Change in sop format generally |