- OBJECTIVE:

To establish a standardized procedure for the microbiological analysis of production personnel gowning to monitor and ensure the cleanliness of gowning and to maintain aseptic conditions in production areas.

- SCOPE:

This SOP applies to all personnel involved in the production process who are required to wear gowns in controlled environments within the pharmaceutical manufacturing facility.

- RESPONSIBILITY/ACCOUNTABILITY:

- Microbiologist conducts the analysis of gowning according to this SOP.

- Quality Control Head reviews the results of the analysis and ensures compliance with this SOP.

- The Quality Manager ensures that personnel adhere to gowning procedures and coordinates sampling with the microbiologist as specified in the SOP.

- ABBREVIATIONS AND DEFINITIONS:

- Gowning: The process of donning specialized clothing and equipment to protect both the product and personnel from contamination in controlled environments.

- CFU: Colony Forming Unit, a measure of viable bacterial or fungal cells.

- PROCEDURE:

Sampling Frequency:

- Perform microbiological analysis of gowning on a routine basis e.g Twice a week or as required by production schedules.

- Additional sampling should be performed following any deviation from standard gowning procedures or contamination event.

Sampling Sites:

The following gowning areas should be sampled:

- Armpit of the gown

- Neck of the gown

- The whole body of the gown

Sample Collection

Swabbing Technique:

- Hold the swab by the handle, avoiding contact with the swab tip to prevent contamination.

- Swab the designated surface area using a consistent, overlapping stroke technique. Typically, swab the armpit of the gown, neck of the gown and the whole body of the gown.

Swab Handling:

- Immediately place the swab into the pre-labeled sterile sampling tube or container.

- For microbial analysis, submerge the swab in a transport medium.

Sample Transport:

- Transport the swab samples to the microbiology laboratory immediately.

- If a delay is unavoidable, store samples at 2-8°C and analyze them within 24 hours to ensure viability.

Microbiological Analysis

Media preparation:

- Suitable media (soya bean digest broth, Eosin methylene blue agar, MacConkey broth, soya bean digest agar and Sabouraud Dextrose Agar) are prepared as per sop BML-QDG-MC-001

- The media is allowed to cool to about 45°C (maximum up to 55°C), but don’t let it solidify.

Note: If the media is prepared already and solidified, melt it by placing it over a water bath or other heat source.

Arrangement of glassware inside the laminar air flow

- Arrange sterile Petri plates, sterile MacCartney bottles and sterile test tubes.

- Label the test tubes and petri plates at the edge of the bottom of the plate with the Media name, Personnel information, analysis date, analysis completion date.

Sample preparation:

- Transfer the swab stick into a test tube containing 9ml of sterile tryptic soy broth medium to re-suspend the microorganisms.

- Perform serial dilutions of the sample if required to achieve countable colony-forming units (CFU).

Method used

Pour-plate method: for isolating and counting the viable microorganisms present in a liquid sample, which is added along with or before molten agar medium prior to its solidification.

Determination of total aerobic microbial count (TAMC) and total combined yeasts and molds count (TYMC) by the pour plate method.

- Slightly open the lid of the petri plate and dispense 1 ml of diluted sample in the center of the Petri plate using a sterile calibrated pipette.

- Open the lid of the prepared media. (soya bean digest agar for TAMC and Sabouraud Dextrose Agar for TYMC)

- Pour about 15 ml of the sterilized molten media at the appropriate temperature above the sample.

- Close the lid of the plate then mix the sample in the media properly by gently swirling the plate. The plate is generally swirled in an “S” or “8” shape.

- Allow the media to completely solidify.

- Simultaneously perform the negative control by plating 1ml of diluent used instead of the sample for each medium.

- Incubate the plate in an inverted position under suitable incubation conditions (For TAMC Incubate the plates of Soybean Casein Digest Agar at 30°C-35°C for 48hours and For TYMC incubate the plates of Sabouraud Dextrose Agar at 20°C - 25°C for 72hours

Test for Absence of Pathogen

Test for E. coli:

- Prepare the sample dilution as described in “determination of TAMC and TYMC”

- Transfer 1ml of the sample into a sterile MacCartney bottle containing 9ml of sterile MacConkey broth.

- Incubate the MacConkey broth bottle at 300C-350C for 24 hours.

- After incubation shake the tube and transfer loop full portion of MacConkey broth on the surface of the pre incubated Eosin Methylene Blue Agar petri plate.

- Incubate plates at 30°C to 35°C for 48 hours.

- For negative control, proceeding the same as above except without sample, streak a portion of incubated sterile MacConkey broth medium on the surface of pre incubated Eosin Methylene Blue Agar.

- Observe and record the presence or absence of specified microorganisms.

Colony Counting:

- After incubation, count the number of colonies on each plate following the sop on how to operate colony counter machine.

Interpretation of Results:

- Record the CFU per personnel into both the infusion gowning analysis record sheet BML-QDG-MC-012-F01 or Syrup gowning analysis record sheet BML-QDG-MC-012-F02 respectively.

- The growth on tryptic soy agar and sabouraud dextrose agar should not exceed limit of 10cfu if any and there must be absence of pathogens.

- Compare the results against the acceptance criteria outlined in the relevant guidelines or internal specifications.

- Report and identify any deviations or unusual results to Quality Control Head.

Corrective Actions:

- If microbial counts exceed acceptance criteria, initiate a deviation report.

- Implement corrective actions, such as retraining personnel, revising gowning procedures, or increasing cleaning frequency.

Precautions

- Ensure all sampling is performed under aseptic conditions to prevent contamination.

- Handle all contact plates and swabs with care to avoid introducing external contaminants.

- Follow biosafety guidelines when handling and disposing of microbial cultures.

Materials and Equipment:

- Sterile Petri-dishes

- Required Microbiological Media

- Sterile Swab Stick

- Incubator

- Laminar flow cabinet

- Bunsen burner

- 70% IPA

- Cotton wool

- Adhesive tape

Safety Precautions

Ensure the use of appropriate personnel protective wear.

- REFERENCE/ENCLOSURE

- Environmental monitoring record sheet for infusion non-clean area - BML-QDG-MC-011-F01

- Environmental monitoring record sheet for syrup non-clean area - BML-QDG-MC-011-F02

- Environmental bioburden surveillance location list for clean area - BML-QDG-MC-011-L01

- TRAINING INVOLVEMENT

Quality Control Head

Microbiologist

8.0 DISTRIBUTION AND CONTROL:

| MASTER COPIES | QUALITY ASSURANCE |
| --- | --- |
| CONTROL COPIES | MICROBIOLOGY LABORATORY |

9.0 REVISION HISTORY

| CURRENT REVISION | SUPERSEDE | REASON FOR REVIEW |
| --- | --- | --- |
| ### 02 | ### 01 | To Update |
| ### 03 | ### 02 | - Inclusion of enclosures format - Change in sop format generally |