- WITHIN THE SCOPE OF THE SOP

| Manufacture of Product (changes to Product, API, or excipients) | - Process (times, mixing, speeds, temperatures, order of addition,) - Removal /replacement/ addition of API or excipient - Formula amounts, overages - Batch size, production scale - Shelf life, expiry - In-process controls - Supplier of API or excipients - Specification, grade of API or excipient - Equipment, critical device - Manufacturing suite - Cleaning methods of processes - Calibration frequency or limits - Shape or dimension of product - Imprints/debossing or other markings on products - Fill weight or volumes - Primary pack (product contact container or closure) – change in supplier, material, pack size or shape |
| --- | --- |
| Quality Assurance | - Specification - Specification limits - Laboratory equipment, test methods or test locations - Regulatory changes - Stability protocol - Sterility protocol - Outsourcing of services |
| Facilities | - Utilities - Building /structural work - Replacements of parts on existing equipment and utilities - Environmental monitoring system |
| Finished Products | - Changes to a final pack e.g. leaflets, labels, cartons - Secondary or tertiary packaging – change in supplier, material, pack size or shape |
| IT System | - Any change to a local or global IT system (software, hardware and firmware) |
| Logistics | - Transportation route e.g. sea to air - Logistic service provider - Distribution or storage e.g. via multi-market warehouse |

- GUIDE TO CHANGE CONTROL MANAGEMENT

| LEVEL 1[unlikely to have detectable impact] |  |  |
| --- | --- | --- |
| CATEGORY | EXAMPLE | REQUIRED ACTION |
| Changes to composition and components | quantitative only (except Immediate Release: colour, flavour, ink; Modified Release: + preservative) | Stability testing: one batch on long-term stability data reported in annual report. |
| Manufacturing – Process Changes | changes to parameters (e.g. mixing times, Autoclaving time, operating speeds) within application/validation ranges | one batch on long-term stability data reported in annual report |
| Manufacturing – Equipment Changes | - Change from non-automated or non-mechanical equipment to automated or mechanical equipment to move ingredients; and - Change to alternate equipment of the same design and operating principles of the same or of a different capacity. | one batch on long term stability |
| LEVEL 2 [could have significant impact] |  |  |
| CATEGORY | EXAMPLE | REQUIRED ACTION |
| Changes to composition and components | quantitative > Level 1, plus any change in excipient grade (Modified Release: + change in excipient specifications | Stability testing, and sterility testing. |
| Manufacturing – Process Changes | changes to parameters (e.g. mixing times, autoclaving time, operating speeds) outside application/validation ranges | stability, sterility |
| Manufacturing – Equipment Changes | change to equipment of different design and different operating principles | stability, sterility |
| Level 3 likely to have significant impact. |  |  |
| Changes to composition and components | quantitative > Level 2, plus addition or deletion of an excipient (except for a colour, flavour, ink) | Requirements for level 3 include stability testing, sterility testing and an in vivo study. |
| Manufacturing – Process Changes | Change in the type of process, such as from sugar drying to direct compounding with the sugar. | stability, sterility, and Bioequivalent study |