- Purpose

- To validate the cleaning procedure for equipments used for manufacturing is effective to remove the ingredients of previous product and detergent used to reduce the microbial contaminants. After use, the equipment is cleaned to prevent cross-contamination of drug products.

- Objective

- To establish a documented evidence of the cleaning effectiveness of the cleaning procedure to a predetermined residue level. The efficacy of cleaning procedure should be such that it effectively removes traces of the previous product and detergent used.

- To verify that the cleaning procedure is able to clean the defined equipment consistently and reliably.

- Responsibilities

- Production Supervisor: To ensure that the various pieces of equipment are cleaned according to the relevant SOP. (SOP no :BML/PR/07,09,10,12,17,20)

To train and monitor the operator for collecting sample by final rinse/swab of the various parts of the equipment.

- Head of Chemical Lab:  Responsible for validating analytical method used to analyze traces of active ingredients of the previous product and traces of detergent used.

To test sample according to validated analytical method

III. Head of Microbiological Lab.

Responsible for validating analytical method used to analyze and count the microbial contamination

IV. Head of Q.A Department

Responsible for writing of the protocol and final report

To supervise the operation to ensure everything is done according to the protocol

V.  Production Manager

Responsible for checking of the protocol and final report

VI. Engineering foreman

Responsible for supporting production personnel during cleaning validation.

- Procedure

- After completion of manufacturing of drug product, cleaning will be carried out by production personnel according to the relevant SOP.

- The production supervisor inspects the equipment visually to assure that it is clean, and fill the inspection form.

- For determining the active residue, the samples are taken according to the following steps:

- Wipe off the cleaned equipment with swab test kit saturated with purified water or methanol (according to the solubility of the API of the previous product) on the critical areas.

- Put the cotton swab into the vial and close the vial with plastic pilfer proof cap.

- Then send the collected sample to the Q.C department for analysis

- For determining detergent residue, the samples are taken according to the following procedure:

- Rinse the clean equipment with purified water

- Collect approx. 500 ml from the final rinse into clean amber glass bottle/ conical flask and close the bottle with the lid/ with foil paper.

- Then send the collected sample to the Q.C department for analysis  ( chemical/ Microbial analysis)

- For determining microbial contaminants, the sample is taken according to the following procedure:

- Open the lid of the Petri-dish plate.

- Put the prepared media on the surface near the cleaned equipment at the critical area.

- Close the Petri-dish plate with the lid immediately.

- Then send the plate to the Microbiological Lab for incubation.

- Acceptance criteria

- The following criteria are established:

- Visual examination should not reveal any evidence of contamination. The equipment will be checked for this and will be recorded.

- Active ingredient of the previous product: Not more than 10 ppm.

- Samples from final rinse have purified water.

- Conductivity of the samples: Not more than 1.3µS.

- Microbial contaminants of the sample: Not more than 25 CFU/25 cm2

- Corrective action (If required)

- If the cleaning validation results show that the equipments are below the acceptance limits or not more than the maximum detectable limit, the cleaning SOP is adequately validated.

- A failure to meet this equipment would necessitate a review of the cleaning procedure of the residual limits. Modification in the procedure to make it more effective, or a change to more realistic limits would again necessitate three successful cleaning of the equipment before the procedure can be considered acceptable.

- Ref . Documents

- SOP for cleaning of the equipments

- Visual inspection form

- Surface sampling form

- Final rinse sampling form

- Monitoring form

- Definitions and Abbreviation

- SOP – Standard operating procedure

- Ppm – part per million

- Cfu – colony forming unit

- Training Requirements

- On relevant sop’s

- On good documentation practice.

9.0     Distribution and Documentation control

- QA Department

- Production unit

- Engineering  unit

- File copy

- Master copy

Annexure I

Cleaning validation visual inspection form:

| Product cleaned |  | Batch No. |  |
| --- | --- | --- | --- |
| Cleaned by |  | Date |  |
| Cleaning SOP No. |  | Date of revision |  |
| Protocol No. |  | Form No. | CVVIOO1 |

| No. | Equipment inspected | Equipment Identification No. | Result |  | Inspected by QA | Date |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Clean | Not clean |  |  |

Remarks: …………………………………………………………………………

Reviewed By: ………………………				Date……………………………..

Annexure II

Cleaning validation surface sampling form:

| Product cleaned |  | Batch No. |  |
| --- | --- | --- | --- |
| Cleaned by |  | Date |  |
| Cleaning SOP No. |  | Date of revision |  |
| Name of the equipment |  | Equipment No. |  |
| Protocol No. |  | Form No. | CVSSOO2 |

| No. | Sample type | Sample location | Sample code | Area (cm2) | Sampled By | Date |
| --- | --- | --- | --- | --- | --- | --- |
| 1. | Microbiological |  |  |  |  |  |
| 2. | Microbiological |  |  |  |  |  |
| 3. | Chemical |  |  |  |  |  |
| 4. | Chemical |  |  |  |  |  |

Remarks: ………………………………………………………………………………………………

Reviewed By: ………………………				Date……………………………..

Annexure III

Cleaning validation final rinse sampling form:

| Product cleaned |  | Batch No. |  |
| --- | --- | --- | --- |
| Cleaned by |  | Date |  |
| Cleaning SOP No. |  | Date of revision |  |
| Name of the equipment |  | Equipment No. |  |
| Protocol No. |  | Form No. | CVFROO3 |

| No. | Sample type | Which Part of the equipment to be use for sampling | Sample code | Quantity | Sampled By | Date |
| --- | --- | --- | --- | --- | --- | --- |

Remarks: ……………………………………………………………………………………………………

Reviewed By: ………………………				Date……………………………..

Annexure IV

Cleaning validation monitoring form:

| Product cleaned |  | Batch No. |  |
| --- | --- | --- | --- |
| Cleaned by |  | Date |  |
| Cleaning SOP No. |  | Date of revision |  |
| Name of the equipment |  | Equipment No. |  |
| Protocol No. |  | Form No. | CVMFOO4 |

| No. | Parameter monitored | Sample location | Result | Monitored By | Date |
| --- | --- | --- | --- | --- | --- |
| 1. | Detergent |  |  |  |  |
| 2. | Rinsing |  |  |  |  |
| 3. | Final rinsing |  |  |  |  |
| 4. | Mop |  |  |  |  |

Remarks: ………………………………………………………………………………………………………

| Reviewed By: ……………………… Date……………………………. |  |
| --- | --- |