- PURPOSE: To lay down a procedure for Handling of Non Conformances.

- Scope: This Standard Operating Procedure is applicable to all departments

- Responsibility

- HODs/ designee of the concerned departments shall be responsible for informing the non-conformances, if observed, in products.

- Head Quality Assurance or designee to investigate and to decide about the disposition of non-conforming in process products or finished products.

- References

- In House

- Definition

Non-conforming product  :     Any product not conforming the established specifications and/ or not acceptable by virtue of its appearance/ physical  state and/ or by virtue of other reasons which  actually are not the part of set or established standards for that material

Technical Committee     :    HODs or designee from the technical departments like QC, QA,                                            and Production designated for investigating or giving opinion on the non-conformances.

- Procedure

- If In-process product or Finished Product is non-conforming, then it shall be quarantined with distinct ‘HOLD’ label and shifted to the designated area under the supervision QA personnel.

- Concerned department shall raise NCR as per BML/QA/NCR/001, mentioning description of non-conforming in process product or finished product.

- NCR shall be sent to QA for further action.

- In case of in-process product or finished product, Head-QA may constitute a technical committee who shall investigate the cause(s) of in process or finished product non conformance.

- Refer Non Conformance Investigation checklist as per as per BML/QA/NCL/001, before further movement of the NCR to different departments.

- Based on the findings of the investigations and/ or comparison with the specifications and / or validation data and / or product development report and / or sound scientific assessment and / or stability data with the product or the working experience with the similar kind of products or molecules, etc. Head-QA and / or technical committee shall then give opinion on the non-conforming in process or finished product that whether the product can be:

    - Use as is: A non conformance may be minor (no significant impact on product’s form or function) and it can be used as is.

    - Rejected: Non conforming products are rejected, when failing in critical or major test parameters that have significant affect on the product quality.

- Concerned department shall give CAPA report, if applicable.

- Following completion of investigation, each NCR shall be assigned a number by officer/ executive Quality Assurance, duly signed and dated. Each Non-conformance report shall have a Nine digit number the form BML/QA/NCR/001, where:

BML 	- 	Denotes

QA       -             Quality Assurance Department

NCR	-	Non-Conformance Report

- - Serial number of the NCR

/	-	“Slash”

- Non conforming products are rejected, when failing in critical or major test parameters that have significant affect on the product quality.

- Based upon the investigation report head quality assurance or QC Head shall give his /her disposition on authorization of destruction of non-conforming in process or finished product.

- After head quality assurance or QC Head disposition, quality assurance personnel shall send a copy of NCR to the concerned departments

- Abbreviation and Definition

SOP	:	Standard Operating Procedure

QA	:	Quality Assurance

NCR                          :     	Non-conformance Report

- Training Requirements

- On relevant SOP’s

- On good cleaning practices

- On cGMP

- On good documentation practice.

10. Distribution:

- All departmental Head .

- Controlled copy

- Master copy