- OBJECTIVE:

This SOP provides guidelines for personnel entering and exiting Non –Clean and Cleanroom environments to minimize contamination risks in manufacturing areas, ensuring adherence to GMP (Good Manufacturing Practices) standards.

- SCOPE:

This SOP applies to all personnel entering or exiting Non –Clean and Cleanrooms within GMP-regulated facilities at BIOMEDICAL Ltd.

- RESPONSIBILITIES/ACCOUNTABILITIES:

- All Employees: Ensure full compliance with the entry and exit procedures for cleanrooms.

- Supervisors: Responsible for monitoring employee adherence and training personnel on cleanroom protocols.

- ABBREVIATION AND DEFINITIONS

GMP- Good Manufacturing Practices

QA – Quality Assurance

LTD- Limited

- PROCEDURES

General:

- Only authorized personnel are allowed into the change room area.  persons who are compromised medically or hygienically (including persons with open wounds, lesions, cough, running nose, conjunctivitis or any other contagious aliments) should not be granted entry into the change room area until medically certified fit.

- Before resuming duty, the medical fitness of each personnel must be submitted to HR.

Entry Procedures

Preparation before Entry

- Remove all jewelry, personal items, and cosmetics that could cause contamination. This includes watches, rings, and bracelets to be kept I a dedicated locker with lock and key.

- Eating and chewing is prohibited in the primary change room

- Wash hands thoroughly with soap and water, followed by hand sanitization using 70%IPA solution at designated hygiene stations.

Primary Change room

- Sanitize with 70%IPA  disinfectant provided.

- Ensure that the hairs, nails are trimmed and clear from cosmetic applications

- Share beard on a regular basis ( for gents)

- Cross over the bench.

- Take the clean factory garments provided; wear the cap first so as to cover the hair completely.

- Wear the factory garments and factory shoe provided

- Check the PPE in the mirror for proper gowning.

- Disinfect hands with70%IPA solution before entering the secondary change room.

NOTE: for the staff working in Packaging, Autoclaving area, Offloading, loading unit need not to enter the secondary change room and should directly go to their various department.

Secondary Change room

- Enter the secondary changing room

- Cross over the cross over bench.

- After changing shoes, proceed into the changing room and take off the company uniform

Donning Cleanroom Garments

- Enter the gowning area in the prescribed order: hairnet, facemask, cleanroom gown, gun booth provided and gloves.

- Ensure the garments are of the correct size and are GMP-approved for cleanroom use.

- Put on sterile gloves and inspect for tears or contamination. Change gloves as necessary.

Entering the Cleanroom

- Upon completion of gowning, pass through the airlock while ensuring doors are not opened simultaneously, as this can disrupt the cleanroom’s positive pressure see SOP No.: BML/PR/032

- Upon entry, sanitize gloved hands using the70% IPA disinfectant before touching any equipment or materials.

Exit Procedures

- Removal of Cleanroom Garments

- Remove gloves, facemasks, and gowns in the correct sequence within the change room. Dispose of single-use items in designated laundry basket provided

- Shoe covers must be removed last and sent to laundry section for proper cleaning

Exiting the Cleanroom

- Pass through the airlock

- Do not open both doors simultaneously, and ensure that the cleanroom remains positively pressurized. see SOP No.: BML/PR/032

- Once outside the cleanroom, sanitize your hands thoroughly and report any contamination incidents to the supervisor immediately.

- Remove the gum booth, clean room garment

- Drop it the laundry basket provided for cleaning.

- Cross over the bench and pass through the buffer area and exit using the control access door.

- Put on the primary garment and the shoe and exit to the primary change room.

Exit from primary change room

- Remove the factory foot wear and primary gowning. Drop it inside the laundry basket provided for cleaning.

- Cross over the bench.

- Put on street clothes and shoes

Monitoring and Compliance

- Supervisors must conduct regular inspections of cleanroom entry and exit practices to ensure compliance with GMP guidelines.

- All deviations from the SOP must be recorded in the deviation log and reported to Quality Assurance (QA) for further action.

Training

- All personnel must receive cleanroom training, which includes proper gowning and entry/exit procedures.

- Refresher training must be conducted every six months.

- Training records must be maintained in the employee training log and be readily available for audit purposes.

- Materials, Equipment, Apparatus and Reagents

- Over all gown/coat

- Head and shoe covers

- Nose mask

- Hand gloves

- Gun booth

- Sterile garment

- REFERENCE AND ENCLOSURE

- ISO 14644: Cleanroom Standards

- Internal SOP for cleanroom gowning and hygiene

7.0 TRAINING INVOLVEMENT

| All departmental managers | All sectional heads |
| --- | --- |
| QA Doc. Unit staff | All staff ( non-clean and clean area) |

8.0 DISTRIBUTION AND CONTROL:

| MASTER COPIES | QUALITY ASSURANCE |
| --- | --- |
| CONTROLLED COPIES | ALL DEPARTMENT |

9.0 CHANGE HISTORY

| CURRENT REVISION | SUPERSEDE | REASON FOR REVIEW |
| --- | --- | --- |
| 00 | Nil | First issue |
| 06 | 05 | - Change in sop format. - Inclusion of traceability of enclosures to parent SOP |