- OBJECTIVE:

This SOP describes the procedure to be followed in the use of Line Clearance for the filling Machine

- SCOPE:

To provide written guidelines for line clearance for Filling Machine.

- RESPONSIBILITIES/ACCOUNTABILITIES:

- Production Officer/Supervisor: Perform initial line clearance and inform QA.

- Quality Assurance (QA): Verify and approve the line clearance before commencement of operations.

- ABBREVIATION AND DEFINITIONS

- Line Clearance: The process of ensuring that all materials, documents, and residues from the previous batch/product have been removed before starting a new activity.

- Batch Record: Document that records all production and quality control data for each batch.

- SOP – Standard Operating Procedure

- QA – Quality Assurance

- BMR – Batch Manufacturing Record

- PPE – Personal Protective Equipment

- cGMP – Current Good Manufacturing Practice

- PROCEDURES

### 6. Procedure

#### When Line Clearance is required

- Before starting a new batch

- Changeover of product or strength

- After major maintenance or equipment breakdown

- After any cleaning activity

  - Steps for Line Clearance

Initial Preparation (Production):

- Clean and sanitize all equipment and area surfaces as per the cleaning SOP.

  - Remove all materials, labels, documents, and waste from the previous batch.

  - Ensure that equipment is labeled as “Cleaned.”

  - Fill out the preliminary section of the Line Clearance Checklist in the BMR

Visual Inspection (Production):

  - Verify absence of previous product residue, unused labels, documents, and tools.

  - Check cleanliness of walls, floors, and ceilings.

Request QA Clearance:

  - Notify QA department after completing the initial checks.

  - QA performs independent verification using the Line Clearance Checklist. In the BMR

QA Verification (QA Officer):

  - Inspect equipment, accessories, and area for cleanliness.

  - Confirm the removal of previous product materials and documents.

  - Verify cleaning records and “Cleaned” status labels.

  - Review environmental conditions if applicable.

Approval and Documentation:

  - If satisfactory, QA signs off the Line Clearance Checklist in the BMR

  - Production may begin operations only after QA clearance.

### Precautions

  - Do not start any process without QA-approved line clearance.

  - Avoid mixing materials or documents from different batches.

  - Use designated bins for rejected or leftover materials.

  - Follow unidirectional flow of personnel and materials to avoid contamination.

  - Ensure that all the personnel wear appropriate wears and follow cGMP rule. See SOP Ref. No BML-QDG-QA-086

  - Ensure the necessary test is done to verify the fitness of the filling machines

  - Check for the filled volume to determine whether under filled or overfilled

- REFERENCE AND ENCLOSURE

ENCLOSURE 1- FILLING LINE CLEARANCE FORM- BML-QDG-QA-022-F01

ENCLOSURE 2 - Cleaning log record of filling room BML/QA/CHK

ENCLOSURE 3 -Clean status labels

7.0 TRAINING INVOLVEMENT

| All departmental managers | All sectional heads |
| --- | --- |
| QA Doc. Unit staff | Filling room staff |

8.0 DISTRIBUTION AND CONTROL:

| MASTER COPIES | QUALITY ASSURANCE |
| --- | --- |
| CONTROLLED COPIES | PRODCUCTION |

9.0 CHANGE HISTORY

| CURRENT REVISION | SUPERSEDE | REASON FOR REVIEW |
| --- | --- | --- |
| 00 | Nil | First issue |
| 06 | 05 | - Change in sop format. - Inclusion of traceability of enclosures to parent SOP |

ENCLOSURE 1

| FILLING LINE CLEARANCE |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Environmental condition Std Temp. : 25±2°C, |  |  |  |  |  |
| Time: |  | Clean |  | Not Clean |  |
| Area |  |  |  |  |  |
| Personnel |  |  |  |  |  |
| Equipment/Vessel |  | Equipment/Vessel No. | Cleaned by | Status | Inspected by QA |
| Semi-Automatic filling machine 1 |  | BL/FR/PE/001 |  |  |  |
| Semi-Automatic filling machine 2 |  | BL/FR/PE/002 |  |  |  |
| Semi-Automatic filling machine 3 |  | BL/FR/PE/003 |  |  |  |
| Semi-Automatic filling machine 4 |  | BL/FR/PE/004 |  |  |  |
| Semi-Automatic filling machine 5 |  | BL/FR/PE/005 |  |  |  |
| Present / Absent of previous product |  | Parameters | specification | Result |  |
| Previous Product: |  | Colour | Colourless |  |  |
| Batch No.: |  | Identification | Must comply |  |  |
| Present Product: |  | pH | 5.0-7.0 |  |  |
| Batch No.: |  | Conductivity | NMT 1.3µs |  |  |
| Remark: |  | QA Officer: |  | Sign/ Date |  |

ANNEXURE 2

CLEANING LOG RECORD FOR FILLING ROOM

DAILY CLEANING WEEK OF _________________SIGNATURE________________DATE_________

| FILLING ROOM./BATCH CODING UNIT | M | T | W | TH | F | REMARKS |
| --- | --- | --- | --- | --- | --- | --- |
| Panels |  |  |  |  |  |  |
| # Floor |  |  |  |  |  |  |
| # Ceiling light covers |  |  |  |  |  |  |
| # Ceilings |  |  |  |  |  |  |
| # Dynamic pass box |  |  |  |  |  |  |
| # Air shower door |  |  |  |  |  |  |
| # Air shower door handle |  |  |  |  |  |  |
| # Stainless Piping |  |  |  |  |  |  |
| # Stainless stood |  |  |  |  |  |  |
| # Stainless bucket |  |  |  |  |  |  |
| # Stainless carts |  |  |  |  |  |  |
| # Light switches |  |  |  |  |  |  |
| # Exhaust lines |  |  |  |  |  |  |
| # Intercom |  |  |  |  |  |  |
| # Digital hygrometers |  |  |  |  |  |  |
| # Wall clocks |  |  |  |  |  |  |
| # CCTV camera |  |  |  |  |  |  |
| # Filling machines/nozzles |  |  |  |  |  |  |
| # Light sockets |  |  |  |  |  |  |
| # Stainless pallet |  |  |  |  |  |  |
| # Stainless buckets |  |  |  |  |  |  |
| # Plastic buckets |  |  |  |  |  |  |
| # Baskets |  |  |  |  |  |  |

Cleaning personnel:                                                                                     Signature                                        Date

Prod. Supervisor   :                                                                                      Signature                                         Date

QA Officer                                                                                                      Signature                                        Date