- OBJECTIVE:

This SOP describes the procedure to be followed Line Clearance in the printing of batch code

- SCOPE:

To provide written guidelines to all necessary precaution and steps taken to have proper Issuance and correct, legible coding.

- RESPONSIBILITIES/ACCOUNTABILITIES:

- Production Officer/Supervisor: Perform initial line clearance and inform QA.

- Quality Assurance (QA): Verify and approve the line clearance before commencement of operations.

4.0 ABBREVIATION AND DEFINITIONS

- Line Clearance: The process of ensuring that all materials, documents, and residues from the previous batch/product have been removed before starting a new activity.

- Batch Record: Document that records all production and quality control data for each batch.

SOP – Standard Operating Procedure

QA – Quality Assurance

BMR – Batch Manufacturing Record

PPE – Personal Protective Equipment

cGMP – Current Good Manufacturing Practice

- PROCEDURES

Production department shall take line clearance from QA in the following condition;

- Product to product change over

- Batch to batch change over.

- Change of primary packing material, printed packing material and changes in code characters

- When coding machine breakdown followed maintenance work.

Pre-Clearance Checks -by the Production Supervisor/Line Checker

- Verify that the previous batch's coding operation is fully complete.

- Ensure the machine status board is updated with details of the completed batch.

- Ensure all excess primary packaging materials (pouches)  and rejects from the previous batch have been removed from the line.

Machine and Area Inspection- by the Production Supervisor/Line Checker

- Visually inspect the coding machine and the surrounding work area for cleanliness and absence of previous batch remnants.

- Clean the encoder, print head, and associated parts of the coding machine thoroughly.

- Ensure all accessories in the area are visibly clean and well kept.

Setup for New Batch - by the Production Supervisor/Line Checker

- Confirm the correct software message (Batch No., Mfg. Date, Exp. Date) is loaded into the coding machine's software

- Conduct a test print to check the quality of the coding on samples and verify it matches the BMR (Batch Manufacturing Record)

Documentation and Approval – by QA personnel

- The Quality Assurance (QA) officer confirm that the line has been cleared and is ready for the next operation before granting approval

- QA personnel in charge should place a "Ready for Use" or similar status tags on the machine, including the new batch's details.

- QA personnel ensure all logbooks and records pertaining to the machine and line clearance is updated and accurate.

- QA personnel must ensure that the printer collects the right PVC pouches for the product and all points of checklists of BMR are filled appropriately

- QA personnel must verify the physical appearance of the pouch (whether de-shaped, bulged or any other) and also verify that a printer sets the Batch No, Mfg. Dates and Expiry Dates on machine clearly.

- Ensure that control samples from the coded batch are properly taken and attached to the Batch Manufacturing Records.

- Make sure the printer prints out samples for verification and the coding record is filled and the sample is attached to the coding/BMR record.

- The coding/BMR record book must be verified by the QA personnel in-charge

- Conduct in-process controls throughout the coding process, including random checks of coded products.

- QA must make sure that the printed pouches are checked after 30minutes interval to make sure there is no mistaken.

- In case of shortage, the reason must be known before collecting more PVC pouches

- In case of excess, the reason must be given by the HOD of the department for excess

- During printing, make sure that the printer sort out of the defected PVC pouches / label as rejects.

- Perform a reconciliation of all used and unused PVC materials to account for every batch

- Document any deviations from the standard procedure in a deviation report and ensure corrective actions are in place.

- REFERENCE AND ENCLOSURE

ENCLOSURE 1- BATCH CODING LINE CLEARANCE FORM- BML-QDG-QA-023-F01

7.0 TRAINING INVOLVEMENT

| All sectional heads | Supervisors/Line leaders |
| --- | --- |
| QA Doc. Unit staff | Batch coding personnel |

8.0 DISTRIBUTION AND CONTROL:

| MASTER COPIES | QUALITY ASSURANCE |
| --- | --- |
| CONTROLLED COPIES | PRODUCTION |

9.0 CHANGE HISTORY

| CURRENT REVISION | SUPERSEDE | REASON FOR REVIEW |
| --- | --- | --- |
| 00 | Nil | First issue |
| 06 | 05 | - Change in sop format. - Inclusion of traceability of enclosures to parent SOP |

ENCLOSURE 1

| BATCH CODING LINE CLEARANCE |  |  |
| --- | --- | --- |
| Previous Product : | B.No: |  |
| Present Product: | B.No: |  |
| Check List | Checked by (Production) | Verified by ( QA ) |
| Area Cleanliness (Floor, walls, ceiling, Machines, Pass box). |  |  |
| Personnel cleanliness |  |  |
| Cleaning log of the batch coding machines |  |  |
| Batch coding machines logbook available and filled |  |  |
| Ensure Proper pouches are taken for coding. |  |  |
| Verify the status cleaned labels |  |  |
| Ensure Proper Coder setting is printed on the pouches. |  |  |
| Pressure differential record |  |  |
| Environmental Conditions : Std Temp. : 25±2°C, Actual Temp. : 0C Std Relative humidity: 65+50C Actual RH: |  |  |