- OBJECTIVE:

The purpose of this procedure is to prove that the equipment cleaning procedure can consistently clean the previous product, the cleaning agent ( if any) and microbial residues to an acceptable level to prevent possible contamination and cross contamination.

To decide the worst –case product for performing cleaning validation for equipment

- SCOPE:

This SOP is applicable for validating cleaning procedures followed for process equipment and accessories used in manufacturing of Biomedical Products.

- RESPONSIBILITIES/ACCOUNTABILITIES:

- Production :cleaning of equipment and accessories as per respective SOPs

- Utility : equipment surface area calculation

- Quality Assurance – sampling and data compilation.

- Quality control: sampling and analysis of samples.

- Production Manager: reviews of the cleaning validation protocol and report.

- Head of QA: review and approval of the cleaning validation protocol and report.

- ABBREVIATION AND DEFINITIONS

SOP – Standard operating procedure

Ppm – part per million

Cfu – colony forming unit

DEFINITION{S)

Cleaning validation (CV) : it is a documented evidence that an approved cleaning procedure is consistent I reducing product residue and removal of cleaning agents ( if any) , bioburden, colour ( if any) from equipment and accessories within the acceptable limit.

Worst case: It identifies the hardest-to-clean product, biggest batch, longest time between cleaning, or most complex equipment to ensure the process succeeds even under stress, guaranteeing safety and efficacy in normal use, noteS

- PROCEDURES

- After completion of manufacturing of drug product, cleaning will be carried out by production personnel according to the relevant SOP.

- The production supervisor inspects the equipment visually to assure that it is clean, and fill the inspection form.

- For determining the active residue, the samples are taken according to the following steps:

- Wipe off the cleaned equipment with swab test kit saturated with purified water or methanol (according to the solubility of the API of the previous product) on the critical areas.

- Put the cotton swab into the vial and close the vial with plastic pilfer proof cap.

- Then send the collected sample to the Q.C department for analysis

- For determining detergent residue, the samples are taken according to the following procedure:

- Rinse the clean equipment with purified water

- Collect approx. 500 ml from the final rinse into clean amber glass bottle/ conical flask and close the bottle with the lid/ with foil paper.

- Then send the collected sample to the Q.C department for analysis  ( chemical/ Microbial analysis)

- For determining microbial contaminants, the sample is taken according to the following procedure:

- Open the lid of the Petri-dish plate.

- Put the prepared media on the surface near the cleaned equipment at the critical area.

- Close the Petri-dish plate with the lid immediately.

- Then send the plate to the Microbiological Lab for incubation.

Acceptance criteria

- The following criteria are established:

- Visual examination should not reveal any evidence of contamination. The equipment will be checked for this and will be recorded.

- Active ingredient of the previous product: Not more than 10 ppm.

- Samples from final rinse have purified water.

- Conductivity of the samples: Not more than 1.3µS.

- Microbial contaminants of the sample: Not more than 25 CFU/25 cm2

Corrective action (If required)

- If the cleaning validation results show that the equipment are below the acceptance limits or not more than the maximum detectable limit, the cleaning SOP is adequately validated.

- A failure to meet this equipment would necessitate a review of the cleaning procedure of the residual limits. Modification in the procedure to make it more effective, or a change to more realistic limits would again necessitate three successful cleaning of the equipment before the procedure can be considered acceptable.

- REFERENCE AND ENCLOSURE

- ENCLOSURE 1- Visual inspection form- BML-QDG-QA-047-FO1

- ENCLOSURE 2-Surface sampling form- BML-QDG-QA-047-FO2

- ENCLOSURE 3-Final rinse sampling form- BML-QDG-QA-047-FO3

- ENCLOSURE 4-Monitoring form- BML-QDG-QA-047-FO4

7.0 TRAINING INVOLVEMENT

| All departmental managers | All sectional heads |
| --- | --- |
| QA Doc. Unit staff | All staff |

8.0 DISTRIBUTION AND CONTROL:

| MASTER COPIES | QUALITY ASSURANCE |
| --- | --- |
| CONTROLLED COPIES | ALL DEPARTMENT |

9.0 CHANGE HISTORY

| CURRENT REVISION | SUPERSEDE | REASON FOR REVIEW |
| --- | --- | --- |
| 00 | Nil | First issue |
| 06 | 05 | - Change in sop format. - Inclusion of traceability of enclosures to parent SOP |

Enclosure 1

Cleaning validation visual inspection form:

| Product cleaned |  | Batch No. |  |
| --- | --- | --- | --- |
| Cleaned by |  | Date |  |
| Cleaning SOP No. |  | Date of revision |  |
| Protocol No. |  | Form No. | CVVIOO1 |

| No. | Equipment inspected | Equipment Identification No. | Result |  | Inspected by QA | Date |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Clean | Not clean |  |  |

Remarks: …………………………………………………………………………

Reviewed By: ………………………				Date……………………………..

Enclosure 2

Cleaning validation surface sampling form:

| Product cleaned |  | Batch No. |  |
| --- | --- | --- | --- |
| Cleaned by |  | Date |  |
| Cleaning SOP No. |  | Date of revision |  |
| Name of the equipment |  | Equipment No. |  |
| Protocol No. |  | Form No. | CVSSOO2 |

| No. | Sample type | Sample location | Sample code | Area (cm2) | Sampled By | Date |
| --- | --- | --- | --- | --- | --- | --- |
| 1. | Microbiological |  |  |  |  |  |
| 2. | Microbiological |  |  |  |  |  |
| 3. | Chemical |  |  |  |  |  |
| 4. | Chemical |  |  |  |  |  |

Remarks: ………………………………………………………………………………………………

Reviewed By: ………………………				Date……………………………..

Enclosure 3

Cleaning validation final rinse sampling form:

| Product cleaned |  | Batch No. |  |
| --- | --- | --- | --- |
| Cleaned by |  | Date |  |
| Cleaning SOP No. |  | Date of revision |  |
| Name of the equipment |  | Equipment No. |  |
| Protocol No. |  | Form No. | CVFROO3 |

| No. | Sample type | Which Part of the equipment to be used for sampling | Sample code | Quantity | Sampled By | Date |
| --- | --- | --- | --- | --- | --- | --- |

Remarks: ……………………………………………………………………………………………………

Reviewed By: ………………………				Date……………………………..

Enclosure 4

Cleaning validation monitoring form:

| Product cleaned |  | Batch No. |  |
| --- | --- | --- | --- |
| Cleaned by |  | Date |  |
| Cleaning SOP No. |  | Date of revision |  |
| Name of the equipment |  | Equipment No. |  |
| Protocol No. |  | Form No. | CVMFOO4 |

| No. | Parameter monitored | Sample location | Result | Monitored By | Date |
| --- | --- | --- | --- | --- | --- |
| 1. | Detergent |  |  |  |  |
| 2. | Rinsing |  |  |  |  |
| 3. | Final rinsing |  |  |  |  |
| 4. | Mop |  |  |  |  |

Remarks: ………………………………………………………………………………………………………

| Reviewed By: ……………………… Date……………………………. |  |
| --- | --- |