- OBJECTIVE:

This SOP provides a standard specification for various types of Personal Protective Equipment, their usage and disposal method.

- SCOPE:

To provide written procedure for various types of Personal Protective Equipment, their usage and disposal method.

- RESPONSIBILITIES/ACCOUNTABILITIES:

User Department , All HOD’S, Production Supervisor, Production Pharmacist, Technical Manager, Q.A Officer, Chemist, Microbiologist, Q.A Manager, Supt. Pharmacist / Factory Manager

- ABBREVIATION AND DEFINITIONS

Nil

- PROCEDURES

- After receiving new safety appliances, it must be stored in store department.

- Store department should check the safety appliances as per the specification in presence of safety Department person.

- Defective appliances should be rejected and returned to the supplier.

- Certification for effectiveness and fitness of safety appliances is to be taken from the vendors.

- After approval from safety/QA officer, store department should update the stock.

Usage of PPE:

- Store department issues the safety appliances to user department on issue slip.

- Note: Department head has to ensure that required safety appliances are available in the department.

- Safety appliances are to be used by personnel as per the requirement of the process or operation at work.

- Safety officer should train the user department persons on correct usage of appliances by Safety department.

- After use, wipe the following PPE with cloth

- Safety goggles

- On line air mask

- Ear muff

- Ear plug

- Safety helmet

After use, wipe the following PPE with water.

- Rubber Hand gloves

- Pressure suit

- PVC apron

Disposal procedure

- Before disposal, wash all the items with water and drain the water to effluent plant.

- The washed items should be disposed off as per the procedure given in the below Table.

| Item | Method of Disposal |
| --- | --- |
| 1) Acid/Alkali proof rubber hand gloves, 2) Surgical hand gloves 3) Cotton hand gloves 4) Foam ear plug 5) Safety belts 6) PVC apron | Cut these items using a suitable scissor. Collect it in a suitable polythene bag and send for scrap. |
| Safety Shoes | Pack it in a suitable polythene bag and send for scrap. |
| 1) Safety goggles Face shield | Detach the glass/acrylic sheet and collect in a suitable polythene bag and send for scrap. |
| 1) Safety helmet | Break these items with a hammer, collect it in a suitable polythene bag and send for scrap |

Maintenance:

- All safety appliances are checked prior to use by user & periodically by safety officer for its suitability and working condition.

- If any of the appliances is found defective, it should be discarded. Organic respirators are discarded after expiry date.

Specification for cotton hand gloves:

| Colour | : | White |
| --- | --- | --- |
| Size | : | 10” to 16” |
| Material | : | Cotton |

Specification for safety shoes:

| Colour | : | Black/ white |
| --- | --- | --- |
| Size | : | 5 No. to 10 No. |
| Material | : | Polyvinyl Chloride |
| Composition | : | Fine finished PVC resistance of acid/alkali (Chemical resistance) material. |

Specification for spectacle type safety goggles:

Type A : 	Spectacle type

Colour: 	Black / Frame

Material: 	Plastic and Polycarbonate.

Composition: Adjustable side frame

Colour: 	White (with chemical splash)

Specification for foam ear plug:

| Design | : | Cup type |
| --- | --- | --- |
| Colour | : | black |
| Composition | : | The micro cells of foam ear plug dampens sound wave and protects the wearer from high pitch damaging noise. |

Specification for Chemical Respirator:

| Composition | : | Lunlung protector respirator, Leak proof fitting on every Face, Light in weight provided with replaceable Spare cartridge, suitable for acid gases, Organic vapour, Ammonia Chlorine and combination thereof. |
| --- | --- | --- |

Specification for Ear Muff:

| Colour | : | Yellow |
| --- | --- | --- |
| Composition | : | Comfortable design for maximum attention and comfort, easily adjustable cushion, padded head band rotating 360°C with long life spring action. |

Specification for PVC Apron:

| Colour | : | white |
| --- | --- | --- |
| Size | : | 24” x 36” |
| Composition | : | PVC and rubber apron protects against acid chemicals used in chemical plants laboratories etc. |
| Material | : | PVC and rubber |

Specification for Safety Helmet:

| Colour |  |  | : | White / Blue |  |
| --- | --- | --- | --- | --- | --- |
| Size |  |  | : | 24” x 36” |  |
| Composition |  | : | Industrial helmet for protection of the head head.Head. |  |  |

- REFERENCE AND ENCLOSURE

IN-HOUSE

7.0 TRAINING INVOLVEMENT

| All departmental managers | All sectional heads |
| --- | --- |
| QA Doc. Unit staff | All staff |

8.0 DISTRIBUTION AND CONTROL:

| MASTER COPIES | QUALITY ASSURANCE |
| --- | --- |
| CONTROLLED COPIES | ALL DEPARTMENT |

9.0 CHANGE HISTORY

| CURRENT REVISION | SUPERSEDE | REASON FOR REVIEW |
| --- | --- | --- |
| 00 | Nil | First issue |
| 06 | 05 | - Change in sop format. - Inclusion of traceability of enclosures to parent SOP |