- OBJECTIVE:

To provide written guidelines for cleaning of dispensing booth and room (RM/PM).

- SCOPE:

This SOP covers the procedure for cleaning of dispensing booth and room (RM/PM)at Biomedical Ltd in both Infusion and syrup plant.

- RESPONSIBILITIES/ACCOUNTABILITIES:

- Raw material Officer – is responsible for ensuring compliance with the SOP.

- Store Unit Head – is responsible for compliance of the SOP.

- QA Head shall be Accountable for implementation of SOP.

- ABBREVIATION AND DEFINITIONS

- %- Percentage

- PPE – personnel protective Equipment

- IPA- Isopropyl Alcohol

- QA- Quality Assurance

- LAF – Lamina Air Flow

- No. – Number

- SOP- Standard Operating procedure

- RM – Raw material

- PM – Packaging material

- PROCEDURES

  - Cleaning Tools and Equipment shall be used as per SOP for Cleaning and Sanitization.

  - The disinfectant shall be used and changed as per the disinfectant schedule provided in the SOP.

CLEANING OF DISPENSING BOOTH AND ROOM:

Type-A Cleaning: Type-A Cleaning is applicable in case of following instances,

  - In between dispensing of two excipients.

Type -B Cleaning: Type-B Cleaning is applicable in case of following instances,

- In-between dispensing of colour/flavoured material.

- After cleaning LAF Prefilters.

- After preventive maintenance of equipment and AHU, Area maintenance activity of the dispensing area.

Type –A Cleaning Procedure:

- Before cleaning the LAF, ensure that all the dispensing tools, containers and Raw material of the previous material were removed.

- LAF and balance should be in running condition during cleaning operation.

- Clean the visible powder inside the dispensing booth by using vacuum cleaner/lint free cloth.

- Clean LAF grills by using vacuum cleaner and lint free cloth.

- After completion of cleaning ensure no traces of the previous material are found inside the LAF.

- Record the cleaning activity as “Type-A Cleaning”.

Type –B Cleaning Procedure:

- Before cleaning the LAF, ensure that all the dispensing tools, containers and Raw material of the previous material were removed.

- Switch off the LAF and balances.

- Clean the entire dispensing booth, grills, balance, side panels, workbench, perforated protective screens, Electrical coils and along with the accessories thoroughly by using vacuum cleaner.

- Wipe the entire inner surface of safe working zone including filter grills with dry lint free cloth followed by 70% IPA.

- Clean the side view glass with Window shine solution followed by dry lint free cloth.

- Dry mop the balance platform with clean lint free cloth.

- Clean the balance platform with 70% IPA.

- Windows/Glasses/Waste bin, S.S. railings shall be cleaned by Window shine/all clean.

- Wall fittings shall be cleaned by Window shine/All clean.

- Cleaning of the floor shall be done by dry moping/Vacuum Cleaner and double bucket wet moping system/floor scrubbing

- The double bucket wet moping system shall be done as per SOP.

- Record the cleaning activity as “Type-B Cleaning”.

- Dispensing booth return air filters shall be cleaned when the manometer/ magnehelic gauge reading reaches the alert level by engineering department as per the SOP.

- Perform cleaning as per instructions provided in ‘Type-B Cleaning’ procedure.

- Record the cleaning activity as “Type-B Cleaning” in the cleaning Log register

FREQUENCY OF CLEANING: Daily

Materials, Equipment, Apparatus and Reagents

- PPE

- Clean towel

- latex gloves

- Sodium hypochlorite

- Mops

- Water

- Vacuum cleaner

- REFERENCE AND ENCLOSURE

IN-HOUSE

7.0 TRAINING INVOLVEMENT

| Quality Assurance Head | Store Unit Head |
| --- | --- |
| QA Doc. Unit staff | Raw material Officer |

8.0 DISTRIBUTION AND CONTROL:

| MASTER COPIES | QUALITY ASSURANCE |
| --- | --- |
| CONTROLLED COPIES | ALL DEPARTMENT |

9.0 CHANGE HISTORY

| CURRENT REVISION | SUPERSEDE | REASON FOR REVIEW |
| --- | --- | --- |
| 00 | Nil | First issue |
| 06 | 05 | - Change in sop format. - Inclusion of traceability of enclosures to parent SOP |