1.0	Purpose:

It is essential that the laboratory has appropriate facilities and suitably qualified staff,

So that correct and reliable results are obtained from all testing procedures, whether chemical, physical, biological or microbiological.  Steps should be taken to validate the reliability of the laboratory’s own systems and test methods. The results must be established in a laboratory that is safe.

2.0	Scope:

This procedure covers the Laboratory system.

3.0	Responsibility:

All Quality Operations personnel must observe good laboratory practice while the Quality Control Manager ensures that Good Laboratory Practice is being observed by all staff.

4.0	References

-		In House

5.0	Materials, Equipment, Apparatus and Reagents

## ASSOCIATED DOCUMENT:

## 1. Cleaning logbook

## 2. Laboratory Workbook

## 3. Certificate of analysis

## 6.0	Procedure

6.1. Premises

- Laboratories should be designed, equipped and maintained to suit the operations to be performed therein. Facilities required include writing area, section for the storage of documents, samples, and reagents and the provision of adequate glass wares.

- Chemical and microbiological laboratories should be separated from each other and from manufacturing area. Separate room may be necessary to protect sensitive instruments from vibrations, electrical interference, humidity, temperature etc.

- Provision should be made for safe storage of waste material awaiting disposal

6.2.   Equipment:

- Laboratory equipment and instruments should be appropriate for the testing Procedures being undertaken.

- Equipment and instruments should be serviced and calibrated at suitably specified interval by an assigned person or organization and readily available records be maintained for each instrument. These records should indicate when the next calibration or servicing is due.

- Written operating instruction for each instrument should be readily available.

- Where practicable, suitable arrangements should be made to indicate failure

Or service of the equipment.

- When necessary, analytical methods should indicate steps to verify that the Equipment is functioning satisfactorily.

6.3.   Cleanliness:

- The laboratory and equipment should be kept clean, in accordance with written cleaning procedures.

- Personnel should wear clean, protective clothing appropriate to the duties being performed.

- The disposal of waste material should be carefully undertaken.

6.4.   Reagents:

- Reagents should be dated upon receipts or preparation.

- Reagents made up in the laboratory should be prepared by person(s) competent to do so following laid down procedures. They must be labelled always, the label indicating the name, date of preparation, concentration, standardization factor, expiry date as well as the initial of the person who prepared the reagent.

- Reference standards prepared must be dated and stored under appropriate conditions.

6.5.   Sampling:

- Samples should be taken in such a manner that they are representative of the batches of materials from which they are taken in accordance with written procedures approved.

- Each sample container must bear a label indicating its contents, the batch number, and the date of sampling, the batch size and the name of the sampler.

- Sampling Procedures should include:

- The method of sampling.

- The equipment to be used.

- The amount of sample to be taken.

- Instructions for any required sub-divisions of samples.

- The type and conditions of sample containers to be used.

- Any special precautions to be observed.

- Cleaning and storage of sampling equipment

6.6   Documentation:

- Its’ purpose is to define the system of control , to reduce the risk of error inherent  in purely oral communication, to ensure that  personnel are instructed in detail of, and follow the procedure concerned and to permit investigation and tracing of defective 	products or any other error which may have occurred.

- Person(s) making entries should do so in clear legible writing and should confirm the entry by adding their initials or signatures.

- Reproduced documents should be clear and legible.

- If an error is made or detected on a document, it should be corrected in such a manner that the original entry is not lost and the correction is initialed and dated also.

- Documents should be kept up to date. Any amendments should be formally authorized and signed. In the case of a permanent amendment, the amended document should be replaced at the earliest opportunity by a newly prepared document.

- An outdated document should be removed from active use, and a copy of the format retained for reference purpose.

- Design of Documents: To facilitate proper and effective use of documents, they should be designed and prepared with care and with particular attention to the following:

- The title, nature and purpose of the document should be clearly stated.

- The way the document is to be used, and by whom, should be clearly stated on the document itself.

- For documents which require the entry of data, adequate space should 	be included for such.

6.7.    Records of Analysis:

- For ease of reference a summary of analytical record requirements are given

as follows:

- Name of product or material and product number.

- Date of receipt and sampling.

- Source of product or material.

- Batch or lot number.

- Indication of tests performed.

- Reference to the method used

- Results.

- Decision regarding release, rejection or other status.

- Signature of the analyst, Quality Control Manager and that of the Quality Assurance

Manager.

- The system of documentation should be such that the history of each batch of product, including the utilization and disposal of raw materials, packaging materials disposal and intermediate, bulk and finished products can be determined.

- In addition to the above records, analysts laboratory work book should contain the 	basic data and calculations from which test results were derived e.g. weighing, 	readings, recorder charts etc.

6.8.   Specifications:

- Specifications approved and authorized should be established for all raw materials, packaging materials, intermediate bulk and finished products.

6.9.   Testing:

- The Quality Control Manager ensures that suitable test methods, validated from R&D   	                   unit in the context of available facilities and equipment, are adopted and utilized.

- Samples should be tested in accordance with the test methods referred to, or detailed in the relevant specifications.  The validity of the results obtained should be checked before the material is released or rejected.

- In-process controls must be carried out and should proceed in accordance with	methods approved by the Quality Assurance Manager.

6.10.   Laboratory Safety:

- Special efforts should be made by personnel to comply with and adhere to recognized safety practices and procedures in order to prevent accidents. Personnel with open lesion must not be allowed in the Laboratory.

- Safety Design Features:    Attention must be directed to the hazards of fire and explosion.

The following safety design features should be given attention:

- Detection of fire and fire alarm systems are a necessity.  Storage rooms 	especially solvent storage areas, and the interior of fume hoods should be 	equipped with fire extinguishers.

- Fire safety doors should be installed as necessary and an escape Exits from each laboratory should be provided.

- iii.	All emergency exits should have well-lit or luminescent exist signs. The illuminator must not depend solely on the main electricity Supply.

- Safety showers and eyewash stations should be provided at strategic locations, and they should be checked frequently for proper functioning.

- Safety Equipment:

- All laboratory personnel are to be provided with adequate protective clothing e.g. coats of overalls, apron, gloves, shoes, safety eye glasses etc. preferably made of inert materials. Laboratory personnel are required to wear them.

- Protective masks or shields, special shoes or shoe covers must be provided, so are caps, fire blankets and gas masks.

- First aid kits should be available and conveniently located.  Working signs must be displayed.  Fume cupboards should be used and their efficiency regularly checked. Pipetting must only be done with pipette fillers.  Removable warning signs should be available for individual laboratory stations e.g. inflammable material in use no naked flame etc.

- Hazardous Materials :( Ref: MSDF)

- Before use any hazardous material, always make reference to material safety data sheet for guidance on proper handling of such materials.

- Special attention should be paid not only to accident prevention but also to the protection of laboratory workers who handle potentially hazardous materials.  Precautions and instructions for handling and disposing of the various types of 	hazardous materials should be provided.

6.11.	Personal Habits and Safe Operating Practices

- The Quality Control Manager should insist on certain safety rules for personnel.  These rules include;

- Foods, sweet, chewing gum, beverages and other edibles must not be consumed in the laboratory.

- Smoking is not permitted in the laboratory.

- Testing of chemicals by tasting is forbidden.

- Hand washing is required after removing protective gloves.

- Pipette fillers must be used for all pipetting operation.

- Protective safety glasses must be worn in any potentially hazardous situation.

- All storage containers for chemicals must be so labeled.  The contents of all unlabeled containers should be discarded.

- Used glassware must be emptied of their content and rinsed with water before being released for regular washing.

- Chipped and cracked glassware must be destroyed.

- All laboratory operations and methods must be reviewed for possible Safety problems regularly.

- Ongoing reactions must not be left unattended to.

- Gas cylinders should only be moved with a suitable hand truck never rolled or dragged along the floor.

- Laboratory visitors should be restricted. If persons are allowed in the laboratory they should be accompanied by a member of staff.

- Good housekeeping practices must be established and followed.

7.0        Training Requirements

- On relevant SOP’s

- On good cleaning practices

- On cGMP

- On good documentation practice.

8.0. Distribution:

- Micro Lab.

- Chemical Lab.

- Controlled copy

- Master copy