- OBJECTIVE:

To provide written guidelines for handling the rejected material.

- SCOPE:

This SOP covers the process for handling of the rejected material

- RESPONSIBILITIES/ACCOUNTABILITIES:

Storekeeper, Q.A Officer, Auditor, Quality Assurance Manager, Supt. Pharmacist/ Factory Manager.

- ABBREVIATION AND DEFINITIONS

## NIL

- PROCEDURES

Source of rejected material would be any of the following:

- Damaged material during material receipt

- Discrepancy noted during receipt of material

- Material rejection during testing

- Material rejected because of the quality event during manufacturing

- Material is expired

- Material no longer required to be used in the facility

- Product return and recall

- Material discrepancy reported during dispensing, manufacturing, or packaging of the batch.

Procedure for rejection handling when the material is rejected during testing:

- During testing, if the material gets rejected, the QC analyst issue a memo and rejection labels according to number of containers.

- On rejecting the material in the system, material stock shall be automatically transferred from quarantine/ under test status to rejected status in material management.

- QC person shall affix the rejection label on rejected material containers.

- Once the QC person affixes the rejected material label, warehouse personnel shall transfer the material to the rejection area and make the necessary entry in the rejected material logbook.

- If Raw material or Packaging material gets rejected, the information shall be shared with the procurement department for financial transactions.

- If the rejected material is raw material, send it to the approved agency that performs destruction activity or notify the agency before the materials is either returned to the suppliers or destroyed at factory premises in a timely manner and in line with the guidelines of the agency.

- If the rejected material is a packing material i.e PVC pouches, Labels, ROPP, cartons, leaflets e.t.c it should not be returned to the supplier. Deface / Crush/ shred / cut into pieces of the rejected packing materials after due information / clearance from the purchase department and the supplier and transferred to scrap yard for sale to the Authorized Scrap Collection Contractor for the plastic scraps.

Procedure for the handling of rejection during material receipt:

- If the material is found damaged/ unacceptable as per material receipt procedure during receipt, the warehouse person will record the observation in the observation sheet and intimate it to the Quality Assurance (QA) department through intimation form. Based on the verification, the QA person shall affix the rejection label on the containers, which will not be in the condition to accept for use.

- Warehouse personnel shall transfer the rejected materials to the rejected material room.

- If material is rejected during receipt of material, record the rejection into the outright rejection form.

- The material can then be returned to the material supplier or destroyed at the factory premises.

Procedure for the handling of rejection during manufacturing or packing operations (online rejection):

- While the manufacturing or packaging process is in progress, various instances where material rejection could happen. For example, defective packaging material observed during operation, the material is not performing as intended, machine setting rejection, material spillage, etc.

- In the above cases, the user department shall initiate the online rejection form and get it approved by the department head and QA. Then, QA will affix the rejection label on materials or material containers.

- User department will submit the material to the warehouse and the online rejection form to store in the rejected material area. However, in case of spillage of raw material, it needs not to transfer to the rejection area. It shall be recorded in respective batch records, and material shall be destroyed as per the material destruction procedure by the manufacturing department.

- For material returned to the warehouse, material movement transactions to be done in the material management.

Procedure for material management for the material which is no longer required to be used:

- The manufacturing, procurement, R&D, and supply chain department shall be informed of material available in the factory, which is no longer required to be used in the factory premises.

- In case any of the stakeholders’ request material transfer to other locations or the probability of material to be used shortly, the decision shall be taken accordingly to retain the material or transfer the material to another site.

- If material is no longer required in any of the above scenarios, it can be destroyed after approval.

- In case any artwork of product has been changed, the material shall be blocked by QA and transferred to the rejected material area.

Procedure for management of expired material:

- In case the material is expired, warehouse personnel shall affix the REJECT label over the available label on the container.

- Warehouse personnel shall transfer the material to the rejection area. Record the details in the rejection logbook.

- Initiate the destruction report and, after approval of the destruction report, destroy the material using the destruction approval form BML/QDG/QA/078/F01

Materials, Equipment, Apparatus and Reagents

- Recording materials

- REFERENCE AND ENCLOSURE

- Destruction approval form - BML/QDG/QA/078/F01

- NAFDAC GMP FOR PHARMACEUTICAL PRODUCTS GUIDELINES 2021, 7.50 -7.55, 3.17b

7.0 TRAINING INVOLVEMENT

| All storekeeper | All sectional heads |
| --- | --- |
| All Manager | All production supervisor |

8.0 DISTRIBUTION AND CONTROL

| MASTER COPIES | QUALITY ASSURANCE |
| --- | --- |
| CONTROLLED COPIES | STORE |

9.0 CHANGE HISTORY

| CURRENT REVISION | SUPERSEDE | REASON FOR REVIEW |
| --- | --- | --- |
| 00 | Nil | First issue |
| 01 | 00 | Due for review |
| 06 | 05 | - Change in sop format. - Inclusion enclosures to parent SOP |