- OBJECTIVE:

To provide the method for packaging of batch of intravenous infusion batches.

- SCOPE:

To provide written guidelines for packaging of batch of intravenous infusion.

- RESPONSIBILITIES/ACCOUNTABILITIES:

Packaging supervisor, Packagers, Production Pharmacists.

- ABBREVIATION AND DEFINITIONS

IPQA – In process Quality Assurance

LDP- Low Density Polyethylene

IPA- Isopropyl alcohol

BMR- Batch Manufacturing Record

- PROCEDURES

Line Clearance

  - Allow the pouches to cool for not less than 8 hours

  - Ensure that the products are cool and clear (not milky).

  - Check that the packaging line being used is clean & free from previous product pouches, sticker labels, rejects, overprinted cartons.

  - Check that the packaging floor & work bench are clean & dry. Clean it if necessary.

  - Check the batch number to be packed & collect the associated BMR.

  - Collect the required packaging materials required for the batch.

  - Intimate the IPQA personnel for the line clearance.

  - The Quality Assurance personnel will check for the line clearance & sign in the specified place in the BMR.

  - Packaging of batch only after due clearance from the Quality Assurance Personnel.

  - On the roller marker for outer carton coding, fix the required digits & alphabets stating product name, batch number, Manufacturing date & Expiry date.

  - Test the print of the coding for outer cartons on plain paper & get it Counter signed by the quality assurance personnel before coding of shipper carton

Separating & Cleaning of Pouches

- After the line Clearance by IPQA, Sanitize your hand with 70% IPA, then hand press to observe for leakages as in SOP

- Clean the pouches with napkin soaked in clean water to which some quantity of IPA has been added

- While cleaning, separate if there are pouches that stick together

- Take the already offloaded and detached pouches on the tray to the packaging line.

- Transfer the cleaned pouches to Leak Testing Machine.

- Perform a leak test on a sample of sealed IV bags to ensure proper sealing and detect leakages

- Ensure that the machine is operated according to the written procedure

- Transfer the products to the Visual Inspection table

- Perform Visual Inspection of pouches for batch no, Manufacturing date, Expiry Date, filled volume and for visible suspended/embedded particles as in SOP BML-PDF-PK-002

- Change Sighters every 30 minutes of visual inspection of the pouches.

- Transfer the well viewed pouches to the nylon bag overwrapping section

Overwrapping

- Put the pouches to LDPS bags with the tube side of the pouch to the inner side of the bag without tearing the bag.

- Transfer the overwrapped pouches to the outer carton.

- Record the number of cartons in BMR.

Batch coding and Labelling

- Coding of outer carton with the required information, including; batch number, Exp. and Mfg. date

- Place correct and appropriate sticker label on the shipper carton

- Ensure labels are legible, accurate and securely attached

- Ensure the correct batch number is used for each packaging cycle

Placing Pouches in Cartons

- Arrange the cartons (overprinted 5-ply corrugated boxes) on the designated table. The

- Production caution label and identification sticker label should be attached.

- Arrange the required number of pouches i.e. 20 pouches for 500ml pack size, 10 pouches for 1000ml pack size and 40 pouches for 100ml except 50% Dextrose Infusion (75 pouches) in shipper carton.

- The arrangement of pouches in the corrugated box should be of uniform pattern and as desired by the pack size and should be placed flatly in the boxes.

- Then seal the cartons.

- Remove the QC Samples from the Batch which should be ideally 10 pouches per batch except 100ml which is 20 pouches.  Place this in corrugated boxes and transfer this to QC sample retention area.

- Transfer and stack on the pallets and off the walls, the finished good cartons in the quarantine area of the Finished Goods Store with designed stacking configuration of 10 rows, 8 columns for 500ml and  10 rows, 9 columns for 1000ml

Documentation

- Fill the required pages of BMR with the details required.

- Record all the details of rejects and packaged quantity in the Packaging Record Logbook LGB/PDF/PK/001/01

Packaging Record Logbook LGB/PDF/PK/001/01.

| Date | S/N | Pack Size | Product Name | Batch No. | Autoclave Reject | Packaging | Particles | Deformed | Under filled | Over filled | Bleached | Start Time | End Time | No. of Worker(s) | Cartons Packed | Units | Name |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

- Fill the Shift Packaging report sheet at the end of the shift.

- Fill the packaging material reconciliation sheet at the end of the shift.

Storage and Handling

- Store packaged IV infusions under appropriate conditions (temperature, light protection) to maintain quality and sterility

- Handle and transport them carefully to prevent damage

Reconciliation

- Do the reconciliation of Packaging Materials collected after completion of Packaging of the batch.

- Any significant deviation regarding package quantity, rejection etc. is to be reported to the packaging supervisor at once and an investigation to be carried out and the justified reason to be stated on the BMR which is to be approved by QC.

General

- Hand over all the rejections of the shift to the personnel concerned responsible for the destruction

- Return all the unused packaging materials to the Storekeeper along with materials return note.

- REFERENCE AND ENCLOSURE

- Packaging Record Logbook - LGB/PDF/PK/001/01

7.0 TRAINING INVOLVEMENT

| Production Supervisor | Production Pharmacist |
| --- | --- |
| Production chemist | Packaging Line leaders |

8.0 DISTRIBUTION AND CONTROL:

| MASTER COPIES | QUALITY ASSURANCE |
| --- | --- |
| CONTROLLED COPIES | PRODUCTION |

9.0 CHANGE HISTORY

| CURRENT REVISION | SUPERSEDE | REASON FOR REVIEW |
| --- | --- | --- |
| 00 | Nil | First issue |
| 01 | 00 | Due for revision |
| 06 | 05 | - Due for revision - Change in sop format. |