- OBJECTIVE

- To provide the method for Visual Inspection of post-sterilized infusion in the Packaging Section.

- SCOPE

- To provide written guidelines for Visual Inspection of Pouches in the Packaging Section

- RESPONSIBILITIES/ACCOUNTABILITIES

- Visual Inspection Operators, Packaging Supervisors, Production Pharmacists and QA inspectors.

- ABBREVIATIONS AND DEFINITIONS

Mfg – Manufacturing

Exp – Expiry

- IPA – Isopropyl Alcohol

- AQL – Acceptance Quality Level

- PROCEDURE

- Sanitize your hands with 70% IPA at the beginning of work.

- Put on gloves

- Sanitize your hands having the hand glove on.

- Check 100% of the products in the batch.

- Verify the luminosity of the sighting boards at the start of work as per SOP BML-QDF-QA-092.

- Pick up two pouches at a time.

- Check the detailing of Batch Number, Mfg and Exp dates if printed correctly. Ensure the coding matches the batch being packed.

- Check for the filled volume to determine whether it is underfilled or overfilled.

- Check for the physical appearance of the pouch (whether it is misshapen or otherwise deformed).

- Ensure the prints on the pouch are not smudged.

- If the pouches are free of these defects, hold it in front of the viewing platform and swirl them gently, ensuring air bubbles are not generated.

- Observe a pouch for 5 seconds in front of the white background panel and another 5 seconds in front of the black background panel to detect any black / white / coloured particle(s).

- Distance of product to the eyes of the inspector must be between 20 cm to 40 cm.

- Distance of the product from the background of the inspection booth must be between 20cm to 30cm.

- Inspection movement shall cover the upper side through to the bottom, looking for fibers, settling particles, micelles and any other foreign substance.

- If any particles are observed, reject the pouch.

- Transfer the sighted product to the second stage inspector who also repeats the inspection.

- Change sighters every 30 minutes of visual inspection of the pouches, and with each rotation the use of IPA to sanitize.

- Inspect each pouch in the batch.

- At the end of the inspection of a batch, count the number of rejected pouches and record in the packaging record logbook LGB/PDF/PK/001/01.

- While 100% inspection is ongoing, the QA inspector and the Packaging Supervisor shall sample post-sighted products based on the AQL 0.65% table below.

- These inspectors shall classify the defects seen into critical, major and minor defects as per Table below;

| Description | Lot size: 3050 /1750 | Sample code: K |  | Lot size: 11250 | Sample code: M | Lot size: 500 | Sample code: H |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Sample size | 125 pouches = 6 cartons |  |  | 315 pouches = 8 cartons |  | 50 pouches = 13 cartons |  |
| Defect Category | Decision |  |  | Decision |  | Decision |  |
|  | Acceptance |  | Rejection | Acceptance | Rejection | Acceptance | Rejection |
| Critical | 0 |  | 1 | 0 | 1 | 0 | 1 |
| Major | 2 |  | 3 | 5 | 6 | 1 | 2 |
| Minor | 10 |  | 11 | 21 | 22 | 5 | 6 |

- If the number of defects observed at this examination is ≥rejection level, the whole batch must be re-sighted.

- Sighting and examination shall be done no more than three times for a batch, after which it must be destroyed following destruction procedures BML-QDG-QA-078.

- Then document number of times sighted, quantity received, rejected and quantity packed in BMR.

Equipment Used:

- Visual Inspection Station with vertical viewing platform with alternate matt-black and non - “glare” white patches with concealed white lamp arrangement.

6. REFERENCE AND ENCLOSURE:

- NIL

7.0 TRAINING INVOLVEMENT

| Production Manager | All Production Pharmacist |
| --- | --- |
| All production sectional heads | All IPQA officers |

8.0 DISTRIBUTION AND CONTROL:

| MASTER COPIES | QUALITY ASSURANCE |
| --- | --- |
| CONTROLLED COPIES | PRODUCTION |

9.0 CHANGE HISTORY

| CURRENT REVISION | SUPERSEDE | REASON FOR REVIEW |
| --- | --- | --- |
| 00 | Nil | - First issue |
| 01 | 00 | - To review |
| 05 | 04 | - Due for review |
| 06 | 05 | - Due for review - Inclusion of rejection level and action to take on such |