- OBJECTIVE:

To provide written guidelines for cleaning and operation of mixing vessels.

- SCOPE:

This SOP covers the procedure for cleaning and operation of mixing Vessels.

- RESPONSIBILITIES/ACCOUNTABILITIES:

Production Supervisor, Production Pharmacist, Quality Assurance Officer.

- ABBREVIATION AND DEFINITIONS

WFI- Water for Injection.

- PROCEDURES

BATCH TO BATCH CLEANING

1.	Ensure that the mixing vessel is empty and close down the valves.

2.	Allow the hot distilled water to flow into the empty mixing tank and collect the hot WFI in a bucket and rinse the inside walls of the mixing tank, stirrer and cover thoroughly.

3.	Drain the water from mixing tank.

PRODUCT TO PRODUCT CLEANING

(Note: Product-to-Product cleaning is required only when the previous product contains any ingredient, which is not present in the next product to be continued).

1.	Ensure that the mixing vessel is empty and close the valves.

2.	Allow the distilled water to flow into the empty mixing tank and collect the distilled water in a bucket and rinse the inside walls of the mixing tank thoroughly using long hose and drain the water from mixing tank.

3.	Ensure that the stirrer and mixing tank covers are cleaned properly.

4.	Drain off the water from mixing tank.

5.	Repeat the steps 2-4 above.

6.	Collect the rinse water from the tank and send to Quality Control Dept. for testing for traces of previous product.

7.	Document the procedure

MATERIALS, EQUIPMENT

WFI

Bucket

- REFERENCE AND ENCLOSURE

NIL

7.0 TRAINING INVOLVEMENT

| Production manager | Production pharmacist |
| --- | --- |
| Sectional heads | All production staff |

8.0 DISTRIBUTION AND CONTROL:

| MASTER COPIES | DOCUMENTATION UNIT |
| --- | --- |
| CONTROLLED COPIES | PRODUCTION DEPARTMENT |

9.0 CHANGE HISTORY

| CURRENT REVISION | SUPERSEDE | REASON FOR REVIEW |
| --- | --- | --- |
| 00 | Nil | First issue |
| 06 | 05 | - Change in sop format. - Inclusion of traceability of enclosures to parent SOP |