- OBJECTIVE:

To provide the methods required in producing a batch of intravenous fluid.

- SCOPE:

To provide written guidelines for preparing a batch of intravenous infusion.

- RESPONSIBILITIES/ACCOUNTABILITIES:

Production Pharmacist, Production Supervisors, Foremen, Operators.

- ABBREVIATION AND DEFINITIONS

S/S= Stainless Steel.

BMR= Batch Manufacturing Record.

- PROCEDURES

PRE-PRODUCTION CHECKS

- Check for the pre-production plan and ensure that the raw materials are dispensed for the batch (follow appropriate requisition procedure).

- Check that the BMR is available and that it has been appropriately stamped and authorized.

- Ensure that the AHU (air handling unit) is operational and that adequate pressure differential is maintained.

- Also ensure that the temperature and relative humidity are within the specified range (22°C-25°C and 40-60% RH respectively).

MANUFACTURING

- Check for the batch number, manufacturing and expiry date(s) coding on the pouches in order to sign the rechecked column of the BMR.

- Ensure the mixing tank is free from the previous product.

- Allow QC to have a sample of the WFI from the collection loop.

- After approval from QC, resume collecting WFI for production.

- Check the weight of the dispensed raw materials and ensure that they are correct. Report any discrepancies to the Pharmacist in charge.

- Check the master formulation method mentioned in the BMR and ensure that there are no modifications authorized.

- Manufacture the batch according to the procedure given, if any.

GOOD CLEAN PRACTICES IN MANUFACTURING

- Ensure that the clean area garments are worn properly and that the nose, mouth and hairs are covered.

- Wear latex hand gloves while weighing raw materials.

- Do not touch the walls and railings of the mezzanine floor while wearing hand gloves, replace hand gloves if this occurs.

- While adding acid or alkali for pH adjustment do so slowly and allow sufficient time to mix and stabilize.

- Always keep the lid of the manhole of the mixing tank closed except when materials are being added.

- Always write down the details required in the BMR immediately after the operation.

- Any spillages of water and/or raw materials should be cleaned instantly.

Materials, Equipment, Apparatus and Reagents

- Release certificates.

- Status Labels.

- BMR.

- REFERENCE AND ENCLOSURE

INHOUSE

7.0 TRAINING INVOLVEMENT

| Production manager | Production pharmacist |
| --- | --- |
| Sectional heads | All production staff |

8.0 DISTRIBUTION AND CONTROL:

| MASTER COPIES | DOCUMENTATION UNIT |
| --- | --- |
| CONTROLLED COPIES | PRODUCTION DEPARTMENT |

9.0 CHANGE HISTORY

| CURRENT REVISION | SUPERSEDE | REASON FOR REVIEW |
| --- | --- | --- |
| 00 | Nil | First issue |
| 06 | 05 | - Change in sop format. - Inclusion of traceability of enclosures to parent SOP |