- OBJECTIVE:

To provide the method for arrangement of pouches in the batch coding room.

- SCOPE:

- To provide written guidelines for arrangement of coded and codeless pouches in the batch coding Room

- RESPONSIBILITIES/ACCOUNTABILITIES:

Batch coding supervisor, Production Pharmacist, Q.A Officer

Production pharmacist, production supervisors, foremen, operators

- ABBREVIATION AND DEFINITIONS

- Q.A.- Quality Assurance

- PROCEDURES

- Ensure that the production plan for the day is confirmed from the production pharmacist so as to inform the pouches to request and the quantity

- Requisition is made from the warehouse using the requisition booklet and signed by appropriate personnel

- Ensure that protective clothing and gloves are worn

- The pouches are passed through the pass box into the batch coding room from the side outside the coding room while the other end inside the coding room is closed

- After filling the pass box with the pouches, the pass box is closed.

- The pass box is then opened from inside the coding room to carry the pouches

- The pouches while wrapped in a protective nylon are placed carefully on the stainless steel floor pallet without touching the floor

- Ensure that the distance of the pouches trays are at least 50cm(0.5m) from the air vent located at the lower third of the wall

- Detach the pouches one after the other and give to the coding machine operators to code

Coded Pouches

- The pouches are removed from the coding machine arranged in group of 5 to 10 pouches

- In the designated area for coded pouches, arrange the pouches in a clean water proof nylon

- Place it on the stainless steel pallet and should be at least 50cm from the wall return vent located at the lower end of the wall.

- Call the appropriate personnel for line clearance before coding the next batch

Materials, Equipment

Stainless Steel Pouch Pallets

- REFERENCE AND ENCLOSURE

In-House

7.0 TRAINING INVOLVEMENT

| All departmental managers | All sectional heads |
| --- | --- |
| QA Doc. Unit staff | All staff |

8.0 DISTRIBUTION AND CONTROL:

| MASTER COPIES | QUALITY ASSURANCE |
| --- | --- |
| CONTROLLED COPIES | ALL DEPARTMENT |

9.0 CHANGE HISTORY

| CURRENT REVISION | SUPERSEDE | REASON FOR REVIEW |
| --- | --- | --- |
| 00 | Nil | First issue |
| 06 | 05 | - Change in sop format. - Inclusion of traceability of enclosures to parent SOP |