- OBJECTIVE:

To provide the instructions for Cleaning the Filling Room

- SCOPE:

To provide written guidelines for cleaning of the Filling Room.

- RESPONSIBILITIES/ACCOUNTABILITIES:

Cleaning officer, Production pharmacist, Quality Assurance officer

- ABBREVIATION AND DEFINITIONS

N/A

- PROCEDURES

- Ensure that all cleaning equipment are clean i.e. soft cloth, buckets, broom, brushes and mops.

- Clean all areas and equipment with clean, soft cloth and mop the ground properly.

- Fill all buckets needed with hot water and add Soap solution into them.

- Scrub the floor and walls thoroughly with the brush and the prepared soap solution.

- After scrubbing, rinse the floor and walls properly with mop to prevent any stagnant water in the filling room.

- Using some disinfectant, (about 40ml of disinfectant in 1 bucket of water (approx. 4L) mop the floors again. Make sure the filling room floors are dry.

- Make sure all equipment in the filling room are dried with the clean, soft cloth provided then wash and dry all the cleaning equipment.

SCHEDULE FOR DISINFECTANTS

| DAY | DISINFECTANT | CONCENTRATION |
| --- | --- | --- |
| MON | DETTOL (Chlorhexidine Solution) | 2.5%v/v |
| TUE | PURIT (Cetrimide Solution) | 2.0% v/v |
| WED | DETTOL (Chlorhexidine Solution) | 2.5%v/v |
| THU | PURIT(Cetrimide Solution) | 2.0% v/v |
| FRI | DETTOL (Chlorhexidine Solution) | 2.5%v/v |
| SAT | PURIT(Cetrimide Solution) | 2.0% v/v |

Materials, Equipment, Apparatus and Reagents

1. Soft cloths

2. Disinfectant

3. Clean Hot water

4. Soap solution

- REFERENCE AND ENCLOSURE

IN-HOUSE

7.0 TRAINING INVOLVEMENT

| All departmental managers | All sectional heads |
| --- | --- |
| QA Doc. Unit staff | All staff |

8.0 DISTRIBUTION AND CONTROL:

| MASTER COPIES | QUALITY ASSURANCE |
| --- | --- |
| CONTROLLED COPIES | ALL DEPARTMENT |

9.0 CHANGE HISTORY

| CURRENT REVISION | SUPERSEDE | REASON FOR REVIEW |
| --- | --- | --- |
| 00 | Nil | First issue |
| 06 | 05 | - Change in sop format. - Inclusion of traceability of enclosures to parent SOP |