- Objective:

To provide the instructions for cleaning of Chemical Laboratory

- Scope:

To provide written guidelines for cleaning of Chemical Laboratory.

3.0 Responsibility/accountability:

Laboratory Assistant, Quality Control Chemist, Quality Assurance Manager, Supt. Pharmacist

4.0 Abbreviations and Definitions

N/A

- Procedure

##### 5.1 General Guidelines

- Ensure all personnel wear appropriate PPE (gloves, mask, lab coat, safety glasses).

- Always use clean and dedicated cleaning tools to prevent cross-contamination.

- Cleaning shall be carried out three times daily and documented.

- Use disinfectants on a rotational basis to prevent microbial resistance.

##### 5.2 Daily Cleaning Steps

- Verify cleaning equipment is clean and in good condition (e.g., soft cloths, mops, buckets, brooms, brushes).

- Clean reagent bottles and shelves with a lint-free soft cloth.

- Dust and clean crevices, corners, and shelf tops using soft cloth or brush.

- Clean work tables, equipment exteriors, and chairs using 2.5% detergent solution followed by wiping with deionized water.

- Remove spots and stains with 70% IPA (Isopropyl Alcohol). Wipe down table tops using alcohol solution.

- Sweep the floor thoroughly using a soft broom.

- Collect debris into designated bins. Empty and disinfect bins daily.

- Mop the entire floor using diluted soap solution and rotational disinfectant (per schedule).

- Rinse floor with clean water to remove detergent residues.

- Clean windows and doors with a clean, damp cloth.

- Clean walls and ceilings weekly using a reach tool or long-handled brush.

- For wall stains, use diluted detergent or alcohol solution as needed.

- All cleaning activities shall be recorded in the Daily Cleaning chart (BML-QDG-QA-006-F01), including date, time, name of cleaner, and supervisor’s initials.

5.3 Disinfectant Rotation Schedule

| DAY | DISINFECTANT | CONCENTRATION |
| --- | --- | --- |
| MON | DETTOL (Chlorhexidine Solution) | 2.5%v/v |
| TUE | PURIT (Cetrimide Solution) | 2.0% v/v |
| WED | DETTOL (Chlorhexidine Solution) | 2.5%v/v |
| THU | PURIT(Cetrimide Solution) | 2.0% v/v |
| FRI | DETTOL (Chlorhexidine Solution) | 2.5%v/v |
| SAT | PURIT(Cetrimide Solution) | 2.0% v/v |

Materials, Equipment, Apparatus and Reagents

- Deionized Water

- Soft Cloths

- Mops

- Buckets

- Soap

6.0 REFERENCE/ENCLOSURE

IN-HOUSE

7.0 TRAINING INVOLVEMENT

| Quality Assurance Manager | Quality Control Head |
| --- | --- |
| QC Analyst | Laboratory Assistant |

8.0 DISTRIBUTION AND DOCUMENTATION CONTROL:

| MASTER COPIES | QUALITY ASSURANCE |
| --- | --- |
| CONTROLLED COPIES | CHEMICAL LABORATORY |

9.0 REVISION HISTORY

| CURRENT REVISION | SUPERSEDE | REASON FOR REVIEW |
| --- | --- | --- |
| ### 05 | ### 04 | To Update |
| ### 06 | ### 05 | - Inclusion of enclosures format - Change in sop format generally |