- OBJECTIVE:

To lay down the procedure for collection, storage and analysis of stability samples of finished products.

- SCOPE:

The scope of this SOP covers the procedure for conducting real time and accelerated stability studies at Biomedical Limited, Ilorin.

- RESPONSIBILITY/ACCOUNTABILITY:

Q.A Officer, Chemist, Microbiologist, Quality Assurance Manager, Supt. Pharmacist/Factory Manager.

- ABBREVIATIONS AND DEFINITIONS:

- PROCEDURE:

Collection of Sample:

- For each new product collection of sample shall be done for first three consecutive batches for stability study under Accelerated temperature and humidity conditions.

- For all products at least one batch per six month in a year shall be kept under long-term stability.

- Quality Assurance officer shall collect the sample as per instructions from Quality Assurance Manager as per stability schedule.

Sample Size:

- Samples shall be collected as intact-marketed pack. The quantity of sample collected shall be in sufficient number depending upon the product and its stability indicating tests. However sufficient samples shall be collected for completion of the stability studies.

Storage of Sample:

- Samples shall be kept at controlled temperature (30+20C) and Relative humidity (65+5%) conditions for Long-term stability study.

- Samples shall be kept at temperature (40+20C) and Relative Humidity (75+5%) for Accelerated Stability Study.

Testing Parameter and Periodicity of Analysis:

- Testing parameters for different products shall be as per the Stability program/Schedule.

- The initial analysis data shall be taken from Certificate of analysis of the respective product and batch, provided the Date-in (T.) of stability study sample is within the period of one month from the date of release of the batch.

- In case the period is more than one month the initial analysis shall be carried out once again at the time of “Date-in”.

- Additional stability indicating tests (other than product release specification) if any, as per stability program, shall be performed at the time of “Date-in” (T0).

- Periodicity of analysis for stability samples shall be as follows:

- Accelerated stability conditions (40+20C/75+5%) = 0,1,2,3 and 6 months.

- Long term Stability Conditions (30+20C/65+5%) = 0, 3, 6, 9, 12, 18, 24, 36 months.

- Long term stability shall be carried out up to the expiry period of the product.

- Long-term stability analysis shall be performed within +7 working days of the due date. Accelerated stability samples shall be analyzed within +5 working days of the due date.

- The sample for stability studies shall be taken within one month from the date of approval for sale of batch. An annual schedule of the batches to be studied for stability and the date of analysis shall be made every January for the year.

- Entry in stability register shall be made accordingly.

Evaluation of Stability Data:

- Evaluation of stability data shall be made twice in a year, Based on evaluation of data, recommendations if any shall be made by Quality Assurance Manager.

- During stability study any major change in physical parameters, pH, Assay, Content uniformity, Investigation shall be done on any affected batch along with all the batches produced at the same time.

- Investigation shall be extended to all ingredients used, manufacturing process, process parameters followed and analysis at different stages.

- Decision on recalling any batch found to be defective and/or along with other batches shall be taken by ED-Technical Services in consultation with Managing Director.

- REFERENCE/ENCLOSURE:

Nil

- TRAINING INVOLVEMENT:

| Quality Assurance Manager | Quality Control Head |
| --- | --- |
| Quality Control Analysts | Laboratory Assistants |

8.0 DISTRIBUTION AND DOCUMENTATION CONTROL:

| MASTER COPIES | QUALITY ASSURANCE/ DOCUMENTATION UNIT |
| --- | --- |
| CONTROLLED COPIES | QUALITY CONTROL LABORATORY |

9.0 REVISION HISTORY:

| CURRENT REVISION | SUPERSEDE | REASON FOR REVIEW |
| --- | --- | --- |
| ### 05 | ### 04 | To Update |
| ### 06 | ### 05 | - Inclusion of enclosures format - Change in sop format generally |