- OBJECTIVE:

To provide written guidelines for cleaning and operation of Melting Point Apparatus

- SCOPE:

This SOP covers the procedure for Melting Point Apparatus

- RESPONSIBILITY/ACCOUNTABILITY:

Quality Assurance Officer, Quality Control Chemist, Quality Assurance Manager and Superintendent Pharmacist.

4.0 ABBREVIATIONS AND DEFINITIONS:

-SOP – Standard operating Procedure

-PPE – Personal Protective Equipment

- PROCEDURE:

#### 5.1 Cleaning Procedure

- Ensure the Melting Point Apparatus is switched off and disconnected from the power source.

- Wipe the exterior surface of the instrument using a soft, clean, dry cloth.

- Inspect the electric cord and ensure it is firmly and properly connected.

- Do not use any solvents or harsh chemicals during cleaning.

- Document the activity in the Melting Point Apparatus Cleaning Logbook (BML-QDF-CL-026-F01).

#### 5.2 Operating Procedure

- Fill the water bath chamber with clean lukewarm water up to ½ – ⅔ of its total capacity.

- Turn the power switch to the “ON” position (marked ‘I’). The power indicator will illuminate.

- Adjust the thermostat knob to the desired working temperature.

- Place the capillary tube containing the sample into the designated holder.

- Monitor the melting process until the sample completely liquefies. Record the melting point accordingly.

- After the test, turn off the power switch and unplug the instrument.

- Store the instrument in a safe location.

- Document the activity in the Melting Point Apparatus Usage Logbook (BML-QDF-CL-026-F02)

Materials, Equipment, Apparatus and Reagents

- PPE

6.0 REFERENCE/ENCLOSURE:

ENCLSOURE 1- Melting Point Apparatus Cleaning Logbook (BML-QDF-CL-026-F01)

ENCLOSURE 2- Melting Point Apparatus Usage Logbook (BML-QDF-CL-026-F02)

7.0 TRAINING INVOLVEMENT

| Quality Assurance Manager | Quality Control head |
| --- | --- |
| Quality Control Analysts | Laboratory Assistants |

8.0 DISTRIBUTION AND DOCUMENTATION CONTROL:

| MASTER COPIES | QUALITY ASSURANCE/ DOCUMENTATION UNIT |
| --- | --- |
| CONTROLLED COPIES | CHEMICAL LABORATORY |

9.0 REVISION HISTORY

| CURRENT REVISION | SUPERSEDE | REASON FOR REVIEW |
| --- | --- | --- |
| ### 05 | ### 04 | To Update |
| ### 06 | ### 05 | - Inclusion of enclosures format - Change in sop format generally |