- Objective:

To provide written guidelines for Operation of Fogger.

- Scope:

This SOP covers the process of Cleaning and Operation of Fogger used in the chemical laboratory

3.0 Responsibility/accountability:

Quality Control Chemist, Quality Assurance Officer, Quality Assurance Manager.

4.0 Abbreviations and Definitions

- cGMP – Current Manufacturing practice

- SOP – Standard operating Procedure

- PPE – Personal Protective Equipment

- Procedure

Preparation and Operation of Fogger

- Ensure appropriate PPE is worn.

- Plug the machine into power source. Extension cord of inappropriate standard is needed in some area.

- Fill the Fogger tank with appropriate disinfectant solution, as per in-house disinfection policy or manufacturer’s recommendation.

- Ensure the disinfectant is compatible with fogger material.

- Turn the machine power switch to ON position using the control switch.

- Wait until the lead light indicator on the control pad comes up (usually after 5minutes), signaling the unit is ready.

- Press and hold the control pad button to start fogging. Fogging is automatically regulated by thermostat which goes off after 3 to 5 minutes.

- Allow the fogger to come up with the lead-light indicator on the control pad before spraying continues.

- Continue spraying until correct amount of fumigant has been dispensed.

- Monitor the room during fogging and restrict access to personnel.

Post-Use Cleaning and Shutdown

- After every use, clean the disinfectant tank of the machine using a hot water.

- Operate the machine for 3 to 5 minutes, flushing the solution through the valves, lines and nozzle.

- Switch off the fogger and disconnect from the power source.

- Wipe the external surfaces of the fogger with a damp cloth.

- Dry all parts and store the fogger in its designated clean equipment area

- Record the Cleaning in the Fogger Cleaning Logbook (BML-QDF-CL-013-F01).

- Record the operation in the Fogger Usage Logbook (BML-QDF-CL-013-F02).

Materials, Equipment, Apparatus and Reagents

- PPE

6.0 REFERENCE/ENCLOSURE

ENLCOSURE 1- Fogger Cleaning Logbook (BML-QDF-CL-013-F01).

ENCLOSURE 2- Fogger Usage Logbook (BML-QDF-CL-013-F02).

7.0 TRAINING INVOLVEMENT

| All departmental managers | All sectional heads |
| --- | --- |
| QA Doc. Unit staff |  |

8.0 DISTRIBUTION AND DOCUMENTATION CONTROL:

| MASTER COPIES | QUALITY ASSURANCE |
| --- | --- |
| CONTROLLED COPIES | ALL DEPARTMENT |

9.0 REVISION HISTORY

| CURRENT REVISION | SUPERSEDE | REASON FOR REVIEW |
| --- | --- | --- |
| ### 05 | ### 04 | To Update |
| ### 06 | ### 05 | - Inclusion of enclosures format - Change in sop format generally |