- Objective:

To provide the procedure for cleaning of work bench in microbiology Laboratory

- Scope:

To provide written guidelines for cleaning of workbench in microbiology Laboratory.

3.0 Responsibility/accountability:

Laboratory Assistant, Microbiologist, Quality Assurance Manager, Supt. Pharmacist

4.0 Abbreviations and Definitions

- IPA: Isopropyl Alchol

- Procedure

- Ensure that all cleaning equipment are properly washed and cleaned i.e. soft cloth, bowls

- Always use the appropriate protective gear. At a minimum, wear latex gloves and goggles.

- Remove loose items from the laboratory workstation. Beakers, test tubes, pipettes etc. should be relocated and washed appropriately.

- Prepare 70% IPA  in a clean bowl  and dip a soft cloth in the mixture and wipe the workbench surface, corners, edges and undersides  thoroughly

- Use a wire brush or other device to remove some residue;

- Caked-on material such as solidified agar or other gelatin-like products can be removed by boiling purified water in the equipment

- An  IPA rinse is useful to sterilize laboratory equipment that requires all microorganisms to be removed before use

- Wash and dry all the cleaning equipment.

Materials, Equipment, Apparatus and Reagents

- Deionized Water

- Soft Cloths

- Bowls

- Mild Soap

- Cotton wool

- 70% IPA

6.0 REFERENCE/ENCLOSURE

IN-HOUSE

- TRAINING INVOLVEMENT

| Quality Assurance Manager | Quality control head |
| --- | --- |
| Quality Microbiology staff | Laboratory assistant |

8.0 DISTRIBUTION AND DOCUMENTATION CONTROL:

| MASTER COPIES | QUALITY ASSURANCE |
| --- | --- |
| CONTROLLED COPIES | QUALITY CONTROL LABORATORY |

9.0 REVISION HISTORY

| CURRENT REVISION | SUPERSEDE | REASON FOR REVIEW |
| --- | --- | --- |
| ### 05 | ### 04 | To Update |
| ### 06 | ### 05 | - Inclusion of enclosures format - Change in sop format generally |