- Objective:

To provide written guidelines for cleaning and Operation of incubators.

- Scope:

This SOP covers the process of cleaning and Operation of Incubators

- Responsibility/accountability:

Laboratory Assistant, Quality Control Chemist, Quality Control Microbiologist, Quality Assurance Manager, Supt. Pharmacist.

4.0 Abbreviations and Definitions

N/A

- Procedure

Cleaning Procedure:

- Disconnect from the power source.

- Open and clean the inner chamber with cotton wool and 70% IPA.

- Clean the outer part of incubators and metal shelves with cotton wool and 70% IPA.

- Close the Fridge door.

- Record the activity in the Incubator Cleaning Logbook (BML-QDF-MC-009-F01).

Operation Procedure

- Connect the switch to main electrical supply.

- Connect the equipment to the main electrical supply

- Set the thermostat to the required temperatures (i.e.22.50C± 2.5oc for fungi growth and 32.50C ± 2.5oc for Bacteria).

- Incubate the plates turn upside down except in special cases.

- Ensure the doors are well closed.

- After successful operation, switch off the appliance and disconnect the cable from the main electrical supply.

- Clean the incubator with 70% IPA when the Incubator assumes room temperature.

- Record the activity in the Incubator Usage Logbook (LGB-QDF-MC-009).

Materials, Equipment, Apparatus and Reagents

- PPE

- Foil paper

- Cotton wool

- 70% IPA

6.0 REFERENCE/ENCLOSURE

ENCLOSURE 1- INCUBATORS CLEANING LOGBOOK- (BML-QDF-MC-009-F01).

ENCLOSURE 2- INCUBATORS USAGE LOGBOOK- (LGB-QDF-MC-009).

- TRAINING INVOLVEMENT

| Quality Assurance Manager | Quality control head |
| --- | --- |
| Quality Microbiology staff | Laboratory assistant |

8.0 DISTRIBUTION AND DOCUMENTATION CONTROL:

| MASTER COPIES | QUALITY ASSURANCE |
| --- | --- |
| CONTROLLED COPIES | QUALITY CONTROL LABORATORY |

9.0 REVISION HISTORY

| CURRENT REVISION | SUPERSEDE | REASON FOR REVIEW |
| --- | --- | --- |
| ### 05 | ### 04 | To Update |
| ### 06 | ### 05 | - Inclusion of enclosures format - Change in sop format generally |