- Objective:

- To serve as a written guide in performing microbiological analysis of raw material

- To establish the microbial fitness, purity or otherwise of raw materials.

- Scope:

- This procedure is to be performed on new stock of raw materials or whenever it deems necessary to do so.

3.0 Responsibility/accountability:

- Microbiologist, Quality control head, Quality Assurance Manager and superintended pharmacist.

- Abbreviations and Definitions

TSB: Tryptone Soya Broth

FTM: Fluid Thioglycollate Medium

TSA: Tryptone Soy Agar

SDA: Sabrouad Dextrose Agar

EMB: Eosin Methylene Blue

- Procedure

- The raw material should be sampled according to this referenced SOP (BML.QDG.CL.005)

- Weigh 1g or 1ml of the sample and dissolve aseptically in 9ml of WFI (usually a 1 in 10 dilution is prepared).

- Pipette an aliquot of 1ml from the stock above in a duplicate sterile Petri dish and add 15 to 20ml of sterile molten TSA, SDA and EMB aseptically around 45oC, swirled and allowed to solidity (pour plate technique) in order to determine the TAMC, TYMC and presence of pathogens in cfu/ml.

- Prepare a negative control plates containing the medium used

- Incubate all the plates in an inverted position at 30 - 35oC for 48 hours for TSA & EMB and at 20 - 25oC for 72 hours for SDA

- Test for Bacterial Endotoxin shall be carried out using the procedure in this SOP (BML-QDF-MC-037)

- Sterility Testing shall be carried out by dissolving 10g or 10ml of the sample in 90ml of WFI, dispense an aliquot of 10ml from the stock and inoculate in the prepared TSB and FTM as per the SOP for Sterility Testing of Finished Product to the (BML.QDF.MC.017)

- Incubate the inoculated bottles i.e TSB at 20-25˚c and FTM at 30-35˚c for seven (7) days.

READING AND INTERPREATION OF RESULTS:

- The bacterial count should not exceed 103cfu/ml while that of the fungal count should not be more than 102cfu/ml and there should be absence of pathogen.

- Visually examine the media bottles daily to its conclusion for macroscopic evidence of microbial growth

REPORTING:

-	The results should be recorded in the microbiological analysis of raw material record (BML-QDF-MC- 015-F01).

-	The final results should be reported to the Quality Control Manager

MATERIALS AND EQUIPMENT:

- Sterile sampling bottle

- 70% IPA

- Paper

- Sterile spatula

- Aluminum foil

- Sterile Analysis bottles

- Pipettes

- Petri dishes

- Cotton wool

- Laminar flow cabinet

- Incubator

- Required microbiological media

6.0 REFERENCE/ENCLOSURE

REFERENCE:

USP<61> Microbiological Examination of Non-sterile products: Microbial Enumeration Tests

USP<85> Bacterial Endotoxins Test

ENCLOSURE 1: Raw Material Record- (BML-QDF-MC-015-F01).

- TRAINING INVOLVEMENT

| Quality Assurance Manager | Quality control head |
| --- | --- |
| Quality Microbiology staff | Laboratory assistant |

8.0 DISTRIBUTION AND DOCUMENTATION CONTROL:

| MASTER COPIES | QUALITY ASSURANCE |
| --- | --- |
| CONTROLLED COPIES | QUALITY CONTROL LABORATORY |

9.0 REVISION HISTORY

| CURRENT REVISION | SUPERSEDE | REASON FOR REVIEW |
| --- | --- | --- |
| ### 05 | ### 04 | To Update |
| ### 06 | ### 05 | - Inclusion of enclosures format - Change in sop format generally |