- Objective:

- To provide written instruction to carry out microbial analysis on PVC Pouches

- Scope:

- To determine the sterility of the PVC pouches

3.0 Responsibility/accountability:

- Microbiologist, Quality Control Head, Quality Assurance Manager, Supt. Pharmacist

4.0 Abbreviations and Definitions

PVC – Polyvinyl Chloride

IPA– Isopropyl Alcohol

- Procedure

- Ensure that the sampling booth, sampling apparatus and containers are clean.

- Take one pack of PVC pouches to be sampled inside sampling booth and place the stopper alongside with the pouch sampled.

- Only one item must be sampled at a time and always make sure that the sampling area is clean, dry and free from previous materials

- Wear hand gloves and nose masks during sampling

- Collect the sample in a clean container and label them with all necessary details e.g. Item description, Batch Number, Date Sampled, Sampled By etc. and transfer to microbiology lab for analysis after appropriate line clearance done.

- Pour 100mls of sterile WFI inside PVC Pouches and cork with the stopper. The rinsing volume should be such that the rinse of the complete inner surface is taken, allow it to stay for 5 minutes.

- Dispense 1ml of the water sample in a duplicate sterile Petri-dish then pour 15 to 20ml of sterile molten TSA, SDA and EMB swirled and allowed to solidify (pour plate technique).

- The plates should be labeled accordingly.

- Incubate the inoculated plates in an inverted position at 20-250c for 72hours for SDA and 30-350c for 48hours for TSA and EMB.

- Sterility Test should be carried out on the PVC pouch following the procedure in this referenced SOP (BML-QDF-MC-014).

Reading and interpretation of results:

- Incubate plates of the different media used as negative control along with the test samples

- The microbial growth should not exceed limit of 10cfu/ml if any, and there must be absent of pathogens.

Reporting:

- The result should be recorded in Microbial analysis for PVC Pouches record (BML-QDF-MC-016-F01)

- The final result should be reported to QAM

- If the specification is exceeded, report immediately to QAM.

Materials, Equipment, Apparatus and Reagents

- Sterile petri-dishes                                                    8.    Aluminum foil

- Lamina flow cabinet

- Bunsen burner                                                          10.   syringe

- Incubator                                                                   12.   Cotton wool

- Adhesive Tape                                                          13. 70% IPA

- Media. i.e. TSA, SDA and EMB

6.0 REFERENCE/ENCLOSURE

REFERENCE:

INHOUSE

USP <71> Sterility test

ENCLOSURE 1: Microbial analysis for PVC Pouches record-(BML-QDF-MC-016-F01)

- TRAINING INVOLVEMENT

| Quality Assurance Manager | Quality control head |
| --- | --- |
| Quality Microbiology staff | Laboratory assistant |

8.0 DISTRIBUTION AND DOCUMENTATION CONTROL:

| MASTER COPIES | QUALITY ASSURANCE |
| --- | --- |
| CONTROLLED COPIES | QUALITY CONTROL LABORATORY |

9.0 REVISION HISTORY

| CURRENT REVISION | SUPERSEDE | REASON FOR REVIEW |
| --- | --- | --- |
| ### 05 | ### 04 | To Update |
| ### 06 | ### 05 | - Inclusion of enclosures format - Change in sop format generally |