- Objective:

To provide written guidelines for Sampling of water for Microbiological Analysis

- Scope:

This SOP covers the process of Sampling of water for Microbiological Analysis.

3.0 Responsibility/accountability:

Microbiologist, Quality Assurance Manager, Superintendent Pharmacist.

4.0 Abbreviations and Definitions

N/A

- Procedure

- Take a clean, dry conical flask of 250ml capacity and sterilize using dry sterilization method by putting it inside oven set at 1100c – 125oc for an hour.

- Take the sterile, plugged flask to the sampling area.

- The nozzle should be flamed by means of flame from burning cotton wool soaked with 70% IPA for at least 30 secs . The cotton wool should have been wrapped on the spatula.

- Open the valve / tap of the sampling point and let the water flow for about 2-3minutes.

- Unplug the flask and collect about 200ml of the water sample in the flask.

- Plug the flask immediately and label it accordingly, mentioning the sampling point and date.

- Carry out microbiological analysis of the water sample as per the procedures and specification.

- Carry out microbiological analysis of the water sample as per the procedures and specification.

Materials, Equipment, Apparatus and Reagents

- Sterile conical Flask (250ml)

- Sampling Basket

6.0 REFERENCE/ENCLOSURE

INHOUSE

7.0 TRAINING INVOLVEMENT

| All departmental managers | All sectional heads |
| --- | --- |
| QA Doc. Unit staff |  |

8.0 DISTRIBUTION AND DOCUMENTATION CONTROL:

| MASTER COPIES | QUALITY ASSURANCE |
| --- | --- |
| CONTROLLED COPIES | ALL DEPARTMENT |

9.0 REVISION HISTORY

| CURRENT REVISION | SUPERSEDE | REASON FOR REVIEW |
| --- | --- | --- |
| ### 05 | ### 04 | To Update |
| ### 06 | ### 05 | - Inclusion of enclosures format - Change in sop format generally |