- OBJECTIVE:

This procedure is to lay down a procedure for proper disposal of used media

- SCOPE:

This procedure is applicable for used media disposal

- RESPONSIBILITY/ACCOUNTABILITY:

- Microbiologists are responsible for the operation and disposal of the culture media.

- The Quality control head ensures the procedures outlined are followed.

4.0 ABBREVIATIONS AND DEFINITIONS

- PPE: Personal Protective Equipment

- PROCEDURE

FOR MICROBIAL MEDIA PLATES

- Wear rubber hand gloves and nose mask

- Remove media content of Petri-dish with the help of spatula and collect in a metal container.

- Autoclave according to the procedure for the operation of autoclave the metal container containing media discard at 15psi, 1210c for 30 minutes.

- After autoclaving, add 5% of sodium hypochlorite solution to the media and dilute the autoclaved contents with tap water and drain into the running tap water.

- Keep the empty glass Petri-dishes for 1hr in 5% sodium hypochlorite solution, then wash the Petri-dish with soap solution and rinse thoroughly with clean water

- Final rinsing of the petri dishes with WFI

- Disinfect the Petri – dishes with 70% IPA

- Dry them in oven(NLT 800c)for re-use

- Disposable plates should be discarded after keeping them for 1hr in 5% sodium hypochlorite.

FOR CULTURED TUBES

- Autoclave the microbial media tubes at 15ps, 1210c for 30 minutes.

- Drain the contents of tubes into the sink under running tap water.

- Keep the empty tubes for 1 hrs in 5% sodium hypochlorite solution, then wash the Petri-dish with soap solution and rinse thoroughly with clean water

- Disinfect the Petri – dishes with 70% IPA

- Dry them in oven(NLT 800c)for re-use

MATERIALS AND EQUIPMENT:

- PPE

- Cultured media Petri-dishes

- Cultured tubes

- 5% sodium hypochlorite solution

- Liquid soap solution

- WFI water

- 70% IPA

- Oven

Safety, Health and Environment Precautions

Wear appropriate PPE

- REFERENCE/ENCLOSURE

USP

- TRAINING INVOLVEMENT

| Quality Control Head | Quality control microbiologist, Laboratory assistant |
| --- | --- |

8.0 DISTRIBUTION AND DOCUMENTATION CONTROL:

| MASTER COPIES | QUALITY ASSURANCE |
| --- | --- |
| CONTROLLED COPIES | QUALITY CONTROL LABORATORIES |

9.0 REVISION HISTORY

| CURRENT REVISION | SUPERSEDE | REASON FOR REVIEW |
| --- | --- | --- |
| ### 00 | ### Nil | First issue |
| ### 02 | ### 01 | To Update |
| ### 03 | ### 02 | - Inclusion of enclosures format - Change in sop format generally |