- OBJECTIVE:

To provide written guidelines for cleaning and Operation of Chemical Laboratory Oven

- SCOPE:

This SOP covers the process for cleaning and Operation of Chemical Laboratory Oven

3.0 RESPONSIBILITY/ACCOUNTABILITY:

- Laboratory Assistance, Quality Control Chemist, will ensure this procedure is follows.

- The Quality Control Head ensures strict adherence to the procedure.

4.0 ABBREVIATIONS AND DEFINITIONS:

- PPE - Personnel Protective Equipment

- Procedure

Cleaning Procedure:

    - Switch off the equipment.

    - Switch off from the mains and unplug.

    - Open the door, empty the content of the oven.

    - Clean the inside of the oven using clean cotton cloth soaked in 1% v/v liquid detergent

    - Clean the outside surface of the oven and control panel with same cloth

Operation Procedure

    - Switch on the precision oven.

    - Turn on the oven by pressing the ON/OFF switch.

    - Press the ACTIVATION KEY for temperature set-point adjustment at the left or right side of the display. This automatically highlights the temperature set value in yellow colour.

    - Using the TURN CONTROL knob, turn to right or left to adjust the set value to the desired temperature (180 0Cfor NLT 30 minutes or 160 0Cfor 2hours).

    - Save the temperature set value by pressing the confirmation key within the TURN CONTROL knob, the display returns to normal, and the oven begins to adjust to the 	defined temperature set value.

    - Monitor the oven (loaded with the glass wares) to ensure that the observed temperature on the display reaches the set value and then, start to time for the required sterilization time.

    - Upon completion of the sterilization time, fill the oven user logbook provided.

Oven User Logbook LGB/QDS/CL/006/01

| Date | Used for | Start time | End time | Total time use | Status | Operator sign | Checked by | Remark |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

    - Turn off the oven using the ON/OFF switch.

    - Allow to cool to room temperature, and then aseptically unload the items for use.

Materials, Equipment, Apparatus and Reagents

    - PPE

    - 1% v/v liquid detergent

Safety, Health and Environment Precautions

- Wear appropriate PPE

- REFERENCE/ENCLOSURE

    - Medfield oven instruction manual.

- TRAINING INVOLVEMENT

| Quality Control Head | Chemical Analyst, Laboratory assistant |
| --- | --- |

8.0 DISTRIBUTION AND DOCUMENTATION CONTROL:

| MASTER COPIES | QUALITY ASSURANCE |
| --- | --- |
| CONTROLLED COPIES | QUALITY CONTROL LABORATORY |

9.0 REVISION HISTORY

| CURRENT REVISION | SUPERSEDE | REASON FOR REVIEW |
| --- | --- | --- |
| ### 00 | ### Nil | First issue |
| ### 02 | ### 01 | To Update |
| ### 03 | ### 02 | - Inclusion of enclosures format - Change in sop format generally |