- OBJECTIVE:

- This SOP describes the process of Handling Chemicals and storage of chemicals.

- SCOPE:

-     This SOP covers all Chemicals in the chemical laboratory and the factory.

3.0 RESPONSIBILITY/ACCOUNTABILITY:

- Laboratory Assistance, Quality Control Chemist, Quality Control Microbiologist will ensure this procedure is follows.

- The Quality Control Head ensures strict adherence to the procedure

4.0 ABBREVIATIONS AND DEFINITIONS:

- PPE - Personnel Protective Equipment’s

- MSDS - Material safety data sheet

- PROCEDURE:

GENERAL

It should be noted that chemicals and reagents with which you work in the chemical and microbiological laboratories can cause burns, injuries, illness and disease. Great care should therefore be taken in handling chemicals, reagents and Medias as well as materials that may have been in contact with chemicals and reagents.

Chemical Handling

- Upon receipt of any chemical, entries are made in the chemical receipt logbook register LGB/QDS/CL/007/01.

| s/n | Date receives | Chemical name | Qty | Supplier’s name | Date opened | Collected by | Witness by | remark |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

- All chemicals must be handled with proper care, based on their characteristic properties.

- All Chemicals and reagents must be properly labeled with appropriate information

- Chemicals, reagents should be handled only by personnel with prior knowledge of its Characteristic and properties.

- No unauthorized person should handle chemicals without the Knowledge of the chemical analyst in charge.

- When handling any chemical or reagent always use the PPE provided, never assume the Identity of any chemical or reagent not properly labeled.

- Highly toxic and corrosive chemicals like conc. acids are to be stored separately from other Chemicals in a safe and secured place.

- Material safety data sheet {MSDS} for all chemicals in the laboratory are made available near the storage cupboard.

- When disposing off any chemical, always follow the instruction in its MSDS, with reference to its properties and level of hazard to the human and the environment.

REQUIRED MATERIALS:

- Personnel Protective Equipment’s (PPE)

- REFERENCE/ENCLOSURE:

- Nil

- TRAINING INVOLVEMENT:

| Quality Control Head | Chemical Analyst |
| --- | --- |

8.0 DISTRIBUTION AND DOCUMENTATION CONTROL:

| MASTER COPIES | QUALITY ASSURANCE |
| --- | --- |
| CONTROLLED COPIES | QUALITY CONTROL LABORATORY |

9.0 REVISION HISTORY

| CURRENT REVISION | SUPERSEDE | REASON FOR REVIEW |
| --- | --- | --- |
| ### 00 | ### Nil | First issue |
| ### 02 | ### 01 | To Update |
| ### 03 | ### 02 | - Inclusion of enclosures format - Change in sop format generally |