- OBJECTIVE:

To provide written guidelines for conductivity meter.

- SCOPE:

This SOP covers the process of operating conductivity meter in the chemical laboratory

- RESPONSIBILITY/ACCOUNTABILITY:

- The chemical analyst must ensure that the equipment is kept clean

- The user reports any equipment malfunction.

- The Quality control Manager ensures external calibration is carried out as scheduled

4.0 ABBREVIATIONS AND DEFINITIONS

- PPE - Personnel Protective Equipment

- PROCEDURE

- Clean the conductivity meter with dried cotton wool.

- Connect the conductivity meter to the main electricity supply.

- Allow the conductivity meter to warm up for about 15 minutes.

- Put the Read / Check “_” Switch in the check “_” mode. Adjust screw knob (on rear panel)

- Select appropriate frequency range as per the requirement.

- Measure the temperature of the sample solution, whose conductivity has to be determined. Set the temperature control knob of the conductivity meter accordingly.

- Dip the conductivity cell in the sample solution; make sure that the platinum electrode is fully dipped in the solution and no air bubbles trapped in the cell.

- Put the Read / Check “_” switch in READ mode. Select proper range on the range selector switch to get maximum reading on display in millisiemens / micro Siemens. Take reading within 10-15 seconds

- Do not mishandle the conductivity cell.

- Record the usage activities in the user logbook LGB/QDS/CL/009/01 for the conductivity meter appropriately.

Conductivity meter User Logbook LGB/QDS/CL/009/01

| Date | Used for | Start time | End time | Total time use | Status | Operator sign | Checked by | Remark |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

- After use, the cell must be thoroughly cleaned with distilled water and drain the water remnant on the cell with soft tissue.

- Disconnect from the main electrical supply after use.

Materials, Equipment, Apparatus and Reagents

- PPE

Safety, Health and Environment Precautions

- Wear appropriate PPE

- REFERENCE/ENCLOSURE

- Jenway conductivity meter instruction manual.

- TRAINING INVOLVEMENT

| Quality Control Head | Chemical Analyst |
| --- | --- |

8.0 DISTRIBUTION AND DOCUMENTATION CONTROL:

| MASTER COPIES | QUALITY ASSURANCE |
| --- | --- |
| CONTROLLED COPIES | QUALITY CONTROL LABORATORY |

9.0 REVISION HISTORY

| CURRENT REVISION | SUPERSEDE | REASON FOR REVIEW |
| --- | --- | --- |
| ### 00 | ### Nil | First issue |
| ### 02 | ### 01 | To Update |
| ### 03 | ### 02 | - Inclusion of enclosures format - Change in sop format generally |