- Objective:

To provide written guidelines for cleaning and operation of Fume Cupboard

- Scope:

This SOP covers the procedure for cleaning and operation Fume cupboard

- Responsibility/accountability:

- The Laboratory attendant clean the fume cupboard.

- The chemical analyst must ensure that the equipment is kept clean

- The user reports any equipment malfunction.

- The Quality control Manager ensures external calibration is carried out as scheduled

4.0 Abbreviations and Definitions

- PPE - Personnel Protective Equipment

- Procedure

Cleaning

- Clean the interior and exterior surfaces periodically

- After spills using deionized water; wipe the areas down a soap solution and rinse
Frequency: After each use

Preparing for the hood work

- Observe noise and air movement to indicate proper operation

- Work must be performed with the sash at or below the mark at which the fume hood was certified

- All work must be performed at least 5 inches inside the hood

- The sash must be lowered completely when not in use

Operation

- Activate the power button

- Keep the sash in the lowest possible position

- Do not block airfoil with equipment, paper towels and absorbent below the sash for effective air flow

- Minimize foot traffic in front of the hood while working

- Keep items in the fume hood to an absolute minimum and only that which is immediately necessary

- Do not use fume hood as general storage locations for chemicals or equipment

- Switch off from the main electricity supply 15 minutes after use

Materials, Equipment, Apparatus and Reagents

- PPE

- Soap

- Demineralized water

Safety, Health and Environment Precautions

- Wear appropriate PPE

- Be prompt to clean any chemical spillage

- REFERENCE/ENCLOSURE

- In-house

- TRAINING INVOLVEMENT

| Quality Control Head | Chemical Analyst |
| --- | --- |

8.0 DISTRIBUTION AND DOCUMENTATION CONTROL:

| MASTER COPIES | QUALITY ASSURANCE |
| --- | --- |
| CONTROLLED COPIES | QUALITY CONTROL LABORATORY |

9.0 REVISION HISTORY

| CURRENT REVISION | SUPERSEDE | REASON FOR REVIEW |
| --- | --- | --- |
| ### 00 | ### Nil | First issue |
| ### 02 | ### 01 | To Update |
| ### 03 | ### 02 | - Inclusion of enclosures format - Change in sop format generally |