- OBJECTIVE:

- To serve as a written guide in performing microbiological analysis of raw material in syrup plant of Biomedical Limited.

- To establish microbial fitness, purity or otherwise of raw materials.

- The test is also to confirm the absence of specified microorganisms [objectionable microorganisms] on raw materials

- SCOPE:

- This procedure is to be performed on new stock of raw materials or whenever it deems necessary to do so.

3.0 RESPONSIBILITY/ACCOUNTABILITY:

- The microbiologist is responsible for following the procedures outlined in this procedure.

- The Quality Control Head is to ensure the procedures outlined are followed.

4.0 ABBREVIATIONS AND DEFINITIONS

- LAF – Lamina Air Flow

- EMB – Eosin methylene broth

- MA -     MacConkey Agar

- IPA -   Isopropyl Alcohol

- MLT – Microbial Limit Test

- TVC - Total viable count

- TYMC - Total combined yeast and mould count

- TAMC - Total Aerobic Microbial count

- PROCEDURE

Preliminary Procedure

- Ensure the sampling booth is clean and free from materials concerned to previous sampling.

- The sample should be taken randomly according to the sampling plan of Raw materials (BML-QDG-QA-060).

- The Raw materials should be sampled into the sterile sampling bottle using the thief sampler or liquid sampler according to the required SOP.

- The sampling bottle should be labelled accordingly. The points from which the sample have been taken should be properly sealed e.g. by means of paper tape or by replacing and tightening of the cap or as appropriate to prevent contamination of the remaining stock.

- The collected sample should be taken to the laboratory and analyzed accordingly.

Actual Procedure:

- Weigh 1g or 1ml of the raw material sample and dissolve aseptically in 9ml sterile soybean casein digest medium prepared as per SOP for media preparation (BML-QDG-MC-001).

- Determine the Total Aerobic Microbial count (TAMC) and Total Yeast and Molds count (TYMC) by using pour plate method.

- Testing to be performed under LAF of specified area.

- Arrange all the required material on the LAF before starting the test.

- Aseptically Pipette 1ml of the solution from above in a sterile Petri dishes

- Add approximately 15-20ml of sterile Soyabean Casein Digest Agar prepared as per SOP( cooled approximately up to 45oC) in two petri plates for TAMC

- Add approximately 15-20ml of sterile Saboraud Dextrose Agar prepared as per SOP( cooled approximately up to 45oC) in two petri- plates for TYMC

- Gently swirl the contents inside the petri-dishes for proper mixing of the sample, allow medium to solidify, taking the precaution of not spilling the media from the plate.

- Simultaneously perform the negative control by plating 1 ml of diluents used instead of the sample for each medium. Add 15-20ml of the media previously maintained not more than 45Oc and allow for solidification.

- Incubate the plates at 30oC to 35OC for 24 -48 hours for soybean Casein Digest Agar for TAMC and plates of Saboraud Dextrose Agar at 200C to 25oC for 72 hours in an inverted position.

Reading And Interpretation of Results:

- The bacterial count should not exceed 103cfu/ml while that of the fungal count should not be more than 102cfu/ml. There should be an absence of fecal coliform.

- If growth occurs in the negative control plate, the test is invalid, failed negative control must be investigated

Reporting:

  - The results should be recorded in the microbiological analysis (MLT) of raw material record BML/QA/STP/RMS/XXX/F03. (where XXX is for the code of such material)

  - The results should be reported to the Quality Control Head.

- REFERENCE/ENCLOSURE

- SOP for media preparation - (BML-QDG-MC-001)

- sampling plan of Raw materials -(BML-QDG-QA-060)

- Microbial Limit Test (MLT) of raw material record form - BML/QA/STP/RMS/XXX/F03

- TRAINING INVOLVEMENT

| Quality Control Head | Quality control microbiologist, Laboratory assistant |
| --- | --- |

8.0 DISTRIBUTION AND DOCUMENTATION CONTROL:

| MASTER COPIES | QUALITY ASSURANCE |
| --- | --- |
| CONTROLLED COPIES | QUALITY CONTROL LABORATORIES |

9.0 REVISION HISTORY

| CURRENT REVISION | SUPERSEDE | REASON FOR REVIEW |
| --- | --- | --- |
| ### 00 | ### Nil | First issue |
| ### 02 | ### 01 | To Update |
| ### 03 | ### 02 | - Inclusion of enclosures format - Change in sop format generally |