- OBJECTIVE

The objective of this SOP is to describe the Operation of Automatic shrink-wrapping machine

- SCOPE

This SOP provides written guidelines to follow the Operation of Automatic shrink-wrapping machine

- RESPONSIBILITY/ACCOUNTABILITY

- IPQC personnel are responsible for ensuring that this SOP is followed for this purpose.

- The operator shall be responsible for the routine and in-depth cleaning and operating of the machine.

- The production supervisor shall monitor the cleaning and operation and give necessary directives for effective cleaning to be achieved.

- Production pharmacists shall ensure the cleaning is carried out and it’s effective

4.0        DEFINITIONS AND ABBREVIATIONS

SOP - Standard Operating Procedure

QC/QA - Quality Control and Quality Assurance

IPA - Iso propyl alcohol

- PROCEDURE

Cleaning

- Clean every part of the machine with napkin moistened with IPA before putting on the machine

- Clean the heater surface with soft wire brush

Set Up

  - Ensure that the Automatic shrink-wrapping machine is cleaned as per procedure described above

  - Check for line clearance approval by QC/QA.

  - Adjust the in-feed conveyor guides based on the carton height. Ensure there is enough margin between the carton and the conveyor guides so that they move easily without getting stuck

  - Using the range adjustment screw, adjust the product backlog detection sensor on the in-feed conveyor to detect the presence of product at different locations.

  - Set the appropriate pusher plate with respect to the height or width of the product matrix. Unscrew the two bolts on the pusher plate to remove the plate and screw back the two bolt after fixing the right plate.

  - Using the main pusher plate adjustment slot, adjust the starting position of the pusher plate

  - Load the film roll on the film roll guides of the machine following these steps

- Loosen the film roll guides as per the width of the film roll

- Load the new film roll, and ensure the center of the film aligns with the center of the product matrix

- Once the roll is aligned, bring the film roll guides back such that the film roll cannot move laterally

- Repeat this process i-iii for the second roll as well

- Once the rolls are loaded, pass the film manually through the fixed and dancing rollers to achieve proper film dispensing

- Join the top and bottom films using the advanced diagnostics for heater testing

  - Set the heater height using the heater height adjustment screw based on the height of the product matrix. Ensure that there is enough room for the product matrix to pass below the heater.

  - Set the temperature of the shrink tunnel and the speed of the tunnel conveyor for the product matrix based on the film thickness.

Operation

- Once all settings have been done and pre-heating is done, start the in-feed conveyor from the Main screen

- Arrange already approved batch coded product matrix on the in-feed conveyor and allow the product matrix to go through the tunnel.

- Observe the product cartons on the conveyor and make sure they can move properly up to the lifting and stacking area

- Ensure that the film is ready for the first product matrix.

- Check that the product cartons are not getting obstructed during in-feed and stacking operation

- Check that the product matrix being transferred from the stacker to the wrapping area flows upright and smoothly.

- Check that the main pusher transfers the product matrix at the appropriate point for an effective film seal to be made

Shutdown

- After completion of wrapping operation, stop the machine operation from the Main screen

- Ensure that product pusher is in its initial (ready) position. Turn off the heaters from the temperature controller

- Ensure that no product is left in the machine.

- Clean the machine according to procedure described above

SAFETY, HEALTH AND ENVIRONMENT PRECAUTIONS

- Check for proper electrical connection. Note that the electrical cabinet is of high voltage and can be a danger to life.

- Never run the machine if any cover or guard door is not installed.

- Do no open the guard doors during production.

- Do not touch the sealing heaters and the areas around them, they can cause severe burns and injuries.

- Always use hand gloves and nose masks during operation

- REFERENCE/ENCLOSURE

- Nil

7.0 TRAINING INVOLVEMENT

| Production Manager | IPQC |
| --- | --- |
| Production Pharmacist | Operators |

8.0 DISTRIBUTION AND CONTROL

| MASTER COPIES | QUALITY ASSURANCE |
| --- | --- |
| CONTROLLED COPIES | PROCESSING |

9.0 REVISION HISTORY

| CURRENT REVISION | SUPERSEDE | REASON FOR REVIEW |
| --- | --- | --- |
| ### 01 | ### 00 | To Update |
| ### 02 | ### 01 | - Inclusion of enclosures format - Change in sop format generally |