- OBJECTIVE

The objective of this SOP is to describe cleaning of Automatic Rotary Piston Filling/Capping machine.

2.0	SCOPE

This SOP provides written guidelines to follow for the cleaning of Automatic Rotary Piston Filling/Capping machine.

3. 0	RESPONSIBILITY/ACCOUNTABILITY

- IPQC personnel are responsible for ensuring that this SOP is followed for the cleaning before usage.

- The operator shall be responsible for the routine and in-depth cleaning of the machine.

- The production supervisor shall monitor the and give necessary directives for effective cleaning to be achieved.

- Production pharmacists shall ensure the cleaning is carried out and it’s effective

4.0 	DEFINITIONS AND ABBREVIATIONS

QA - Quality Assurance

SOP - Standard Operating Procedure

DM - Demineralized water

5.0        PROCEDURE

Batch To Batch Cleaning

- Ensure that no product liquid is left inside the float tank.

- Fill the float tank with purified water and start the machine without bottles on the conveyor.

- The water will be drawn by the syringes and will be delivered through the filling nozzles.

- Run the machine with QA released demineralized water so that the syringes are cleaned thoroughly.

- Clean the conveyor with clean moist cloth.

- Clean the other parts of the machine with moist cloth and for next batch.

## Product To Product Change Over Cleaning

  - Ensure that no product liquid is left inside the float tank.

  - Fill the float tank with QA released DM water and run the machine, so that the water is drawn by the syringes and delivered through filling nozzles.

  - Remove the syringes from their assembled positions

  - Remove the nozzles for cleaning and disinfection.

  - Wash the syringes and the Teflon with mild soap solution (12.5% sodium laureth sulphate)

  - Fix the syringes in their positions on the machine ensuring no leakage

  - Fill the float tank with hot water (60 - 70°C) and run the machine, to flush the machine through the nozzles until all the water in the float tank run out. Repeat this three times and then fourth time with room temperature DM water.

  - Fix already cleaned and disinfected nozzles.

  - Clean the other parts of the machine, conveyor etc., with DM water and wipe with clean cloth.

  - After thorough cleaning, fill DM water in the float tank and collect the DM water rinse by running the machine through nozzles.

  - Observe the rinse water in a glass beaker visually for clarity. It should be clear and colorless.

  - Intimate the Quality Control to perform in-process checks.

  - The rinse water should be free from traces of previous product.

  - Repeat the cleaning procedure if necessary.

  - Dry wipe the cleaned parts with clean cloth and put clean equipment status label.

SAFETY, HEALTH AND ENVIRONMENT PRECAUTIONS

  - Keep the mains “OFF” while cleaning the machine.

  - Do not allow water to go inside the electrical connections.

  - Use clean cloth for cleaning.

- REFERENCE/ENCLOSURE

- Nil

7.0 TRAINING INVOLVEMENT

| Production Manager | IPQC |
| --- | --- |
| Production Pharmacist | Operators |

8.0 DISTRIBUTION AND CONTROL

| MASTER COPIES | QUALITY ASSURANCE |
| --- | --- |
| CONTROLLED COPIES | FILLING ROOM |

9.0 REVISION HISTORY

| CURRENT REVISION | SUPERSEDE | REASON FOR REVIEW |
| --- | --- | --- |
| ### 01 | ### 00 | To Update |
| ### 02 | ### 01 | - Inclusion of enclosures format - Change in sop format generally |