- OBJECTIVE:

The objective of this SOP is to describe the Operation of Automatic rotary bottle washing machine

2.0	SCOPE

This SOP provides written guidelines to follow the Operation of Automatic rotary bottle washing machine

3.0	RESPONSIBILITY/ACCOUNTABILITY

- The operator will carry out the operation according to this SOP.

- The production supervisor shall monitor and give necessary directives for effective usage of the machine

- Production pharmacists will ensure the operation is carried out and it’s effective

4.0 	DEFINITIONS AND ABBREVIATIONS

DM - Demineralized

SOP - Standard Operating Procedure

PPE - Personal Protective equipment

5.0 	PROCEDURE

SETUP AND OPERATION

- Ensure that the Rotary Bottle Washing Machine is cleaned.

- Check for line clearance approval by QC/QA as per SOP.

- Ensure that the machine is cleaned thoroughly before starting the operation.

- Fill tank I and tank II with DM water.

- Ensure that the flexible pipe connections are tight and proper.

- Ensure that the washing pressure is not less than 3kg/sq.cm.

- Switch ON the main motor, Pump I, Pump II and heater.

- De-carton the bottles from the pack and arrange the bottles on the turn table.

- Ensure the hot water temperature is between 55-60oC

- Wash the bottles in one cycle by passing hot and cold water and air simultaneously to wash the inner and the outer part of the bottle accordingly.

- The washed bottles are delivered through the conveyor into baskets kept at the end of the conveyor.

- Repeat the procedure for batch completion.

SHUTDOWN

- After completion of washing operation switch “OFF” the machine & close all valves for Water line.

- Drain the water from the Tank I and II and ensure that no water is left inside the tank

- Clean Rotary Bottle Washing Machine as per SOP. No.-BML/PR/SYP/010 & put a proper Status Label.

SAFETY, HEALTH AND ENVIRONMENT PRECAUTIONS

- Check for proper electrical connection & water connections are in place & supplies are received.

- Watch for trip hazard when transferring the washed bottle to the designated spot

- Use appropriate PPE

- REFERENCE/ENCLOSURE

- Nil

7.0 TRAINING INVOLVEMENT

| Production Manager | IPQC |
| --- | --- |
| Production Pharmacist | Operators |

8.0 DISTRIBUTION AND CONTROL

| MASTER COPIES | QUALITY ASSURANCE |
| --- | --- |
| CONTROLLED COPIES | PROCESSING |

9.0 REVISION HISTORY

| CURRENT REVISION | SUPERSEDE | REASON FOR REVIEW |
| --- | --- | --- |
| ### 01 | ### 00 | To Update |
| ### 02 | ### 01 | - Inclusion of enclosures format - Change in sop format generally |