- OBJECTIVE:

To provide written guidelines for daily cleaning and sanitation of syrup manufacturing.

- SCOPE:

This SOP provides written guideline to follow for daily cleaning of syrup manufacturing plant.

- RESPONSIBILITY/ACCOUNTABILITY

Production Operators, Production Supervisor, Production Pharmacists, Quality Control officer.

- ABBREVIATIONS AND DEFINITIONS

Nil

- PROCEDURE

CLEANING OF THE AREA

- Dust all the area viz. corridors, corridors, production floor, finished goods store, raw material store, bottle store and packaging material store.

- Remove cobwebs with the aid of a long broom

- First sweep with sweeping brush

- Clean all the windows and doors with glass cleaner with liquid wash and soft napkin.

- Prepare 2%v/v soap solution and mop the area with it using clean mop brush.

- Frequently wash the mop with clean water to remove the dirt collected in it.

SANITATION

- Prepare the disinfectant solution as per the schedule and of the specified concentration.

- Apply the disinfectant solution on the whole floor

- Leave it like this for at least 5 minutes. Mop away the excess disinfectant solution from the floor after 5 minutes using clean water.

FREQUENCY OF CLEANING

- Clean the production floor, corridors at least three times in a shift

- Clean the raw material store twice a shift

- The packaging material store and bottle is to be cleaned daily.

- The boiler room, water treatment area is to be cleaned twice a shift.

MAINTAINING THE CLEANINGNESS OF AREA

- Ensure there is no accumulation of water, oil or grease on the floor

- Be prompt to clean any water spillage

- Any area if dirty, clean at once at that time only irrespective of the schedule.

SCHEDULE FOR DISINFECTANTS

| DAY | DISINFECTANT | CONCENTRATION |
| --- | --- | --- |
| MONDAY | DETTOL(Chlorhexidine solution) | 2.5%v/v |
| TUESDAY | HYPOCHLORITE SOLUTION | 2.0%V/V |
| WEDNESDAY | DETTOL(Chlorhexidine solution) | 2.5%v/v |
| THURSDAY | HYPOCHLORITE SOLUTION | 2.0%V/V |
| FRIDAY | DETTOL(Chlorhexidine solution) | 2.5%v/v |
| SATURDAY | HYPOCHLORITE SOLUTION | 2.0%V/V |

- REFERENCE/ENCLOSURE

DAILY CLEANING CHART- BML-QDG-QA-006-F01

7.0 TRAINING INVOLVEMENT

| Production operator | All sectional heads |
| --- | --- |
| IPQC |  |

8.0 DISTRIBUTION AND CONTROL

| MASTER COPIES | QUALITY ASSURANCE |
| --- | --- |
| CONTROLLED COPIES | PRODUCTION |

9.0 REVISION HISTORY

| CURRENT REVISION | SUPERSEDE | REASON FOR REVIEW |
| --- | --- | --- |
| ### 01 | ### 00 | To update |
| ### 02 | ### 01 | - Inclusion of enclosures format - Change in sop format generally |