- Objective:

To provide written guidelines for cleaning and sanitization of clean area.

- Scope:

This SOP describes the process of cleaning and sanitization of clean area.

3.0 Responsibility/accountability:

Production Supervisors, Production Pharmacists, Quality Assurance Manager    and Superintendent Pharmacist.

4.0 Abbreviations and Definitions

N/A

- Procedure

CLEANING OF THE AREA

- Broom the floor with plastic brushes/wipers to clean away the rejected stoppers, pouch necks etc.

- Prepare 2.5%v/v Soap Solution in Deionised Water & mop the whole area clean.

- Take extra effort to clean the covings between wall & floor as these are the areas prone to accumulation of more dirt.

- Clean all the glasses with liquid glass cleaner & plastic mope.

- Clean the surface of the Tanks, Machines and Mezzanine Floor with wet lint free duster at least once in a shift.

- Clean the return air grills with wet lint free duster at least once in a shift.

- Once in a day in the morning shift clean all the foot wares of the clean area & soak them in disinfectant solution for 15 minutes before using it again.

DISINFECTION OF THE AREA

- Prepare the required disinfectant solution as per the schedule in proper concentration. Use deionised water or distilled water for diluting the disinfectant.

- With the help of the spray pump spray the disinfectant solution on the whole floor.

- Leave it like this for at least 10 minutes. Mop away the excess disinfectant solution from the floor after 10 minutes to render the floor dry.

- Spraying of wall & ceiling is to be done when the lids of manufacturing & storage tanks are closed & when filling operation is not going on. Leave at least 10 minutes time between spraying of wall & ceilings & start of the filling operation.

- Once in a shift pour some disinfectant solution to the drain points after cleaning them.

FREQUENCY OF CLEANING

- Being a classified area the Clean Area requires constant cleaning & disinfections in order to prevent growth of microorganisms in the area.

- At no instance of time there should be water or product accumulation on the floor.

- Be prompt to remove any dirt/debris from the operations like used Raw Material bags, paper tags, labels if on the floor.

- Very frequently clean the bins used to contain the pouches’ cut necks & paper waste.

SCHEDULE FOR DISINFECTANTS

| DAY | DISINFECTANT | CONCENTRATION |
| --- | --- | --- |
| MON | DETTOL (Chlorhexidine Solution) | 2.5%v/v |
| TUE | PURIT (Cetrimide Solution) | 2.0% v/v |
| WED | DETTOL (Chlorhexidine Solution) | 2.5%v/v |
| THU | PURIT(Cetrimide Solution) | 2.0% v/v |
| FRI | DETTOL (Chlorhexidine Solution) | 2.5%v/v |
| SAT | PURIT(Cetrimide Solution) | 2.0% v/v |

Materials, Equipment, Apparatus and Reagents

- PPE

- Hand gloves

- Nose Mask

- REFERENCE/ENCLOSURE

In House

- TRAINING INVOLVEMENT

| All departmental managers | All sectional heads |
| --- | --- |
| QA Doc. Unit staff |  |

8.0 DISTRIBUTION AND DOCUMENTATION CONTROL:

| MASTER COPIES | QUALITY ASSURANCE |
| --- | --- |
| CONTROLLED COPIES | ALL DEPARTMENT |

9.0 REVISION HISTORY

| CURRENT REVISION | SUPERSEDE | REVIEW DATE | REASON FOR REVIEW |
| --- | --- | --- | --- |
| ### 02 | ### 01 | ### July 2022 | To update |
| ### 03 | ### 02 | ### July 2024 | - Inclusion of enclosures format - Change in sop format generally - To update |