- Objective:

This SOP describes the process of for gowning and use of change room

- Scope:

This SOP covers the written guideline for gowning and use of change room

3.0 Responsibility/accountability:

Production staff, production supervisor, production pharmacist, QA officer, QA Manager

4.0 Abbreviations and Definitions

N/A

- Procedure

MAIN ENTRANCE AND CHANGING ROOM

- Change from street clothes into company uniform in the cloak room.

- Upon arrival at the main entrance to the clean room area, the clean room attendant will b e on hand to provide a change of shoes BEFORE entering the main doors

- After changing shoes proceed into the changing room and take off company uniform and put on the clean room uniform, including the nose mask and work boots.

- Using the wash hand basin provided wash hands up to the elbows and dry them using the hand dryer.

- Proceed to the clean room corridor through the air shower. Ensure that during the 10 seconds period in the air shower that your hands are well spread

FILLING ROOM STAFF

- Pass through the second air shower exactly as before and put on gloves

- In the filling room, ensure that you observe the following hygiene etiquette in addition to the filling room SOP.

- Talking is discouraged, if the need arises, please keep voices low

- If any injuries are sustained, alert a supervisor immediately.

- Keep exits out of the filling room to the barest minimum and never re-enter the area with contaminated apparel.

- Do not carry mobile phones on your person

- Keep your nose mask and gloves on at all times.

BATCH CODING ROOM STAFF

- Once in the batch coding room staff should put on their gloves

- Ensure that SOP guidelines are strictly adhered to at all times.

EXIT FROM CLEAN AREA

- Exit the clean room area through the air lock doors at the end of the clean room corridor.

- Once in the air lock compartment, ensure that the doors are closed behind you to maintain the air pressure differential.

- Outside the doors change from the clean room boots into the work shoes provided

Materials, Equipment, Apparatus and Reagents

- PPE

- Hand gloves

- Nose Mask

- REFERENCE/ENCLOSURE

In House

- TRAINING INVOLVEMENT

| All departmental managers | All sectional heads |
| --- | --- |
| QA Doc. Unit staff |  |

8.0 DISTRIBUTION AND DOCUMENTATION CONTROL:

| MASTER COPIES | QUALITY ASSURANCE |
| --- | --- |
| CONTROLLED COPIES | ALL DEPARTMENT |

9.0 REVISION HISTORY

| CURRENT REVISION | SUPERSEDE | REVIEW DATE | REASON FOR REVIEW |
| --- | --- | --- | --- |
| ### 02 | ### 01 | ### July 2022 | To update |
| ### 03 | ### 02 | ### July 2024 | - Inclusion of enclosures format - Change in sop format generally - To update |