- Objective:

This SOP describes the process lay down procedure to establish capability of analyst to perform analysis accurately.

- Scope:

This SOP is applicable to establish capability of analyst to perform analysis accurately.

3.0 Responsibility/accountability:

Trainee Analyst, Quality Assurance Officer, Microbiologist, Quality Control Chemist, Quality Assurance Manager and the Superintendent Pharmacist/Factory Manager

4.0 Abbreviations and Definitions

N/A

- Procedure

- Samples of known analytical values earlier determined by the Quality Assurance Manager are used for analyst competence test

- The analytical value(s) of sample(s) along with acceptable limit(s), shall recorded by the QA Manager and kept away from the analyst

- Standard Test Procedure for each sample will be provided to each analyst.

- No repeat of test will be allowed by the analyst

- The coded samples along with necessary information required for analysis shall be given to the analyst for testing

- Result from the analyst shall be compare with previous results of the coded samples from the QA Manager

- The analyst shall be validated for either one or more of following areas of analysis. 
1. Assay 
2. pH
3. Melting point 
4. Identification Test

- The validation shall include one or more of the following methods of analysis: 
a.   Titrimetric

- Polarimetery analysis

- UV-VIS Spectrometry

- pH Measurement

- HPLC

- Moisture content

- Melting point

- After completion of test the QA Manager will check the test result and compared with expected values.

- The capability of the analyst to perform test shall be considered satisfactory if the results reported by the analyst are within the acceptable limits.

- The analyst shall be revalidated after a period of one year or if a new method of analysis is developed.

- The details like calculations, along with comments of Q.A. Manager shall be filed in training file of analysts.

- New analyst shall be validated within nine months of joining.

- QA Manager will review validation report and will decide for the selection of analyst for routine analysis. If analyst is failed to qualify the test analyst will undergo training on the testing equipment and test procedure.

- In case of three or more analysts are under evaluation then % Relative Standard Deviation will be taken for interpretation of the result. The % RSD should not be more than 1%.

- Fill the result in analyst validation report.

REQUIRED MATERIALS:

- Note book

- Standard Testing Procedure

- Official Pharmacopeias

- Coded samples

- Computer System

- REFERENCE/ENCLOSURE

N/A

- TRAINING INVOLVEMENT

| All departmental managers | All sectional heads |
| --- | --- |
| QA Doc. Unit staff |  |

8.0 DISTRIBUTION AND DOCUMENTATION CONTROL:

| MASTER COPIES | QUALITY ASSURANCE |
| --- | --- |
| CONTROLLED COPIES | ALL DEPARTMENT |

9.0 REVISION HISTORY

| CURRENT REVISION | SUPERSEDE | REVIEW DATE | REASON FOR REVIEW |
| --- | --- | --- | --- |
| ### 00 | ### Nil | ### Nil | First |
| ### 01 | ### 00 | ### Jan 2024 | - Inclusion of enclosures format - Change in sop format generally |