- Objective:

To lay the procedure for the calibration of the weighing balance.

- Scope:

This SOP covers the procedure for the calibration of the Analytical Weighing Balance

- Responsibility/accountability:

Laboratory Assistants, QA/QC analyst, the Quality Assurance Manager and the Superintendent Pharmacist.

4.0 Abbreviations and Definitions

- GLP: Good Laboratory Practice

- GDP: Good Documentation Practice

- Procedure

- Plug the power line to a supply outlet, turn on the power switch and press the power on key.

- Wait till the balance shows 0.000g, and preheat the balance for 30 minutes.

- Clear the weighing pan and press the ‘C’ (calibration) key.

- It displays ‘C’ and ‘g’. (If ‘CE’ is displayed, press ‘T’ key to zero the equipment).

- Load calibration weight unto pan, and wait till the value is logged.

- Record the reading for the calibration.

- Frequency for accuracy: calibrate balance every day, and as when required when drift is noticed.

MATERIALS, EQUIPMENT, APPARATUS AND REAGENTS:

- Standard weights

SAFETY HEALTH AND ENVIRONMENTAL PRECAUTIONS: PPE

- REFERENCE/ENCLOSURE

Equipment Manual

- TRAINING INVOLVEMENT

| All departmental managers | All sectional heads |
| --- | --- |
| QA Doc. Unit staff |  |

8.0 DISTRIBUTION AND DOCUMENTATION CONTROL:

| MASTER COPIES | QUALITY ASSURANCE |
| --- | --- |
| CONTROLLED COPIES | ALL DEPARTMENT |

9.0 REVISION HISTORY

| CURRENT REVISION | SUPERSEDE | REVIEW DATE | REASON FOR REVIEW |
| --- | --- | --- | --- |
| ### 00 | ### Nil | ### Nil | First |
| ### 01 | ### 00 | ### Jan 2024 | - Inclusion of enclosures format - Change in sop format generally |