- Objective:

This SOP describes the process of Sanitization of factory floor drains.

- Scope:

-     This SOP covers all the cleaning and sanitization of all factory floor drains and waste water

Collection loops.

3.0 Responsibility/accountability:

- Production supervisor / foreman, Chemist, Microbiologist, Production Pharmacist, Quality Control Manager, Total Quality Manager and the Superintendent Pharmacist

4.0 Abbreviations and Definitions

N/A

- Procedure

GENERAL:

- All drains in the factory are marked and locations of each drain are noted. Solid waste must not be passed through the marked drains, in case of any drain blockage the production supervisor or foreman shall report such to the technical department and the production pharmacist.

- All drains must be covered properly to prevent entry of insects and rodents into the factory premises.

Sanitization Procedure

- All drains mouth is open and properly cleaned with WFI water and detergent Solution.

Prepare a 5% w/v solution of sodium hypochlorite.

- Pour about one liter of the hypochlorite solution through the funnel down the drains, starting with the one most far from the drain exit point.

- Allow the sanitizing solution to remain in the drains for at least one hour, before rinsing the drains by pouring WFI water into it.

- Ascertain the effectiveness of the sanitization process by swabbing the surface of the drains near the opening, of the cleaned and sanitized drains.

- The process is deem satisfactory if the microbial load reported, is zero for the sterile area and less than 102 cfu for the Mixing and dispensing area and less than 103 cfu for the other areas.

- Repeat the process after 8 days if the microbial report is not satisfactory.

REQUIRED MATERIALS:

-	Personnel Protective Equipments (PPE), Cleaning brush, Plastic buckets and plastic funnel.

- REFERENCE/ENCLOSURE

ENCLOSURE 1- BML-QDS-QA-007-F01

- TRAINING INVOLVEMENT

| All departmental managers | All sectional heads |
| --- | --- |
| QA Doc. Unit staff |  |

8.0 DISTRIBUTION AND DOCUMENTATION CONTROL:

| MASTER COPIES | QUALITY ASSURANCE |
| --- | --- |
| CONTROLLED COPIES | ALL DEPARTMENT |

9.0 REVISION HISTORY

| CURRENT REVISION | SUPERSEDE | REVIEW DATE | REASON FOR REVIEW |
| --- | --- | --- | --- |
| ### 02 | ### 01 | ### July 2022 |  |
| ### 03 | ### 02 | ### July 2024 | - Inclusion of enclosures format - Change in sop format generally |

ENCLOSURE 1

| Date | Drains Location | Material used | Sanitized by | Checked by QA |
| --- | --- | --- | --- | --- |
|  | Mixing unit | Hypochlorite/dettol |  |  |
|  | Filling unit | Hypochlorite/dettol |  |  |
|  | Loading unit | Hypochlorite/dettol |  |  |
|  | Offloading unit | Hypochlorite/dettol |  |  |
|  | Male cloakroom | Hypochlorite/dettol |  |  |
|  | Female cloakroom | Hypochlorite/dettol |  |  |
|  | Water treatment hall | Hypochlorite/dettol |  |  |
|  | Mixing unit | Hypochlorite/dettol |  |  |
|  | Filling unit | Hypochlorite/dettol |  |  |
|  | Loading unit | Hypochlorite/dettol |  |  |
|  | Offloading unit | Hypochlorite/dettol |  |  |
|  | Male cloakroom | Hypochlorite/dettol |  |  |
|  | Female cloakroom | Hypochlorite/dettol |  |  |
|  | Water treatment hall | Hypochlorite/dettol |  |  |
|  | Mixing unit | Hypochlorite/dettol |  |  |
|  | Filling unit | Hypochlorite/dettol |  |  |
|  | Loading unit | Hypochlorite/dettol |  |  |
|  | Offloading unit | Hypochlorite/dettol |  |  |
|  | Male cloakroom | Hypochlorite/dettol |  |  |
|  | Female cloakroom | Hypochlorite/dettol |  |  |
|  | Water treatment hall | Hypochlorite/dettol |  |  |
|  | Mixing unit | Hypochlorite/dettol |  |  |
|  | Filling unit | Hypochlorite/dettol |  |  |
|  | Loading unit | Hypochlorite/dettol |  |  |
|  | Offloading unit | Hypochlorite/dettol |  |  |
|  | Male cloakroom | Hypochlorite/dettol |  |  |
|  | Female cloakroom | Hypochlorite/dettol |  |  |
|  | Water treatment hall | Hypochlorite/dettol |  |  |