- OBJECTIVE:

This procedure refers to line clearance for the packaging hall, floor, packaging table

- SCOPE:

To provide written guidelines for line clearance of packaging area.

- RESPONSIBILITIES/ACCOUNTABILITIES:

The Packaging Line leader/ Officer verify all clearance points on the checklist.

The Packaging supervisor reviews the completed checklist.

The Quality Assurance (QA) personnel conduct a final verification and sign off on the line clearance checklist, authorizing the start of the next batch.

- ABBREVIATION AND DEFINITIONS

Nil

- PROCEDURES

- Make sure all the personnel are in appropriate overall.

- Make sure the packaging hall is clean and tidy.

- Ensure that the packaging materials of the previously packed product are removed.

- Ensure that all the required packing materials are available.

- Ensure that the status board is displayed with the updated batch details

- Ensure that the equipment used in the packaging area is cleaned.

- Ensure that all the logbooks pertaining to the machine are updated.

- Ensure that preventive maintenance of the machines is carried out as per the schedule and the equipment is affixed with preventive maintenance label.

- Ensure that the packing materials required for batch are issued by warehouse and received by production

- Check the coded Cartons for details of coding and if it tally’s with the batch details on the product for packing.

- Check the batch manufacturing record for correctness of batch and product.

- Ensure the activity is started within the same shift in which the line clearance is given by IPQA and if activity is not started within the same shift, then verification of the line clearance activity shall be done before starting the activity in the subsequence shift.

- Ensure there are no leakages on the product.

- Ensure that the finished product is properly rapped and packed inside the outer carton

- Make sure that the production personnel followed the written procedure as prescribed in the individual Batch Manufacturing Record (BMR).

- In case of any observation which is not acceptable or a non-conformance (i.e. product/Material/document pertaining to previous product, incomplete/wrong documentation, and particle) inform the concern personnel on duty, initiate incident and complete the investigation along with CAPA.

- REFERENCE AND ENCLOSURE

ENCLOSURE 1-PACKAGING LINE CLEARANCE FORM- BML-QDG-QA-025-F01

7.0 TRAINING INVOLVEMENT

| Production pharmacist | Production Supervisor/Line leaders |
| --- | --- |
| Quality Assurance Officer | Packagers |

8.0 DISTRIBUTION AND CONTROL:

| MASTER COPIES | QUALITY ASSURANCE |
| --- | --- |
| CONTROLLED COPIES | ALL DEPARTMENT |

9.0 CHANGE HISTORY

| CURRENT REVISION | SUPERSEDE | REASON FOR REVIEW |
| --- | --- | --- |
| 00 | Nil | First issue |
| 06 | 05 | - Change in sop format. - Inclusion of traceability of enclosures to parent SOP |