- OBJECTIVE:

The purpose of this procedure is to prove that the equipment cleaning procedure can consistently clean the previous product, the cleaning agent (if any) and microbial residues to an acceptable level to prevent possible contamination or cross contamination.

To decide the worst –case product for performing cleaning validation for equipment.

- SCOPE:

This SOP is applicable for validating cleaning procedures followed for process equipment and accessories used in manufacturing of Biomedical Products.

- RESPONSIBILITIES/ACCOUNTABILITIES:

- Production Personnel: Cleaning of equipment and accessories as per respective SOPs.

- Production Supervisor: Supervision of the cleaning of equipment and accessories.

- Utility Personnel: Equipment surface area calculation

- Quality Assurance Officer: Sampling and data compilation.

- Quality Control Analyst: Sampling and analysis of samples.

- Production Manager: Reviews of the cleaning validation protocol and report.

- Quality Assurance Manager: Review and approval of the cleaning validation protocol and report.

- ABBREVIATION AND DEFINITIONS

SOP – Standard operating procedure

ppm – part per million

Cfu – colony forming unit

DEFINITION{S)

Cleaning validation (CV): It is a documented evidence that an approved cleaning procedure is consistent reducing product residue and removal of cleaning agents (if any) , bioburden, colour (if any) from equipment and accessories within the acceptable limit.

Worst case: It identifies the hardest-to-clean product, biggest batch, longest time between cleaning, or most complex equipment to ensure the process succeeds even under stress, guaranteeing safety and efficacy in normal use.

- PROCEDURES

- After completion of manufacturing of drug product, cleaning will be carried out by production personnel according to the relevant SOP.

- The production supervisor inspects the equipment visually to assure that it is clean, and fill the inspection form.

- For determining the active residue, the samples are taken according to the following steps:

- Wipe off the cleaned equipment with swab test kit saturated with purified water or methanol. (according to the solubility of the API of the previous product) on the critical areas.

- Put the cotton swab into the vial and close the vial with plastic pilfer proof cap.

- Then send the collected sample to the Q.C department for analysis.

- For determining detergent residue, the samples are taken according to the following procedure:

- Rinse the clean equipment with purified water.

- Collect approx. 500 ml from the final rinse into clean amber glass bottle/ conical flask and close the bottle with the lid or foil paper.

- Then send the collected sample to the Q.C department for analysis (chemical/microbial analysis).

- For determining microbial contaminants, the sample is taken according to the following procedure:

- Open the lid of the Petri-dish plate.

- Put the prepared media on the surface near the cleaned equipment at the critical area.

- Close the Petri-dish plate with the lid immediately.

- Then send the plate to the Microbiological Lab for incubation.

Acceptance criteria

The following criteria are established:

- Visual examination should not reveal any evidence of contamination. The equipment will be checked for this and will be recorded.

- Active ingredient of the previous product: Not more than 10ppm.

- Samples from final rinse have purified water.

- Conductivity of the samples: Not more than 1.3µS.

- Microbial contaminants of the sample: Not more than 25 Cfu/25cm2.

Corrective action (If required)

- If the cleaning validation results show that the equipment are below the acceptance limits or not more than the maximum detectable limit, the cleaning SOP is adequately validated.

- A failure to meet this equipment would necessitate a review of the cleaning procedure of the residual limits. Modification in the procedure to make it more effective, or a change to more realistic limits would again necessitate three successful cleaning of the equipment before the procedure can be considered acceptable.

- REFERENCE AND ENCLOSURE

- ENCLOSURE 1- Visual inspection form- BML-QDG-QA-047-FO1

- ENCLOSURE 2-Surface sampling form- BML-QDG-QA-047-FO2

- ENCLOSURE 3-Final rinse sampling form- BML-QDG-QA-047-FO3

- ENCLOSURE 4-Monitoring form- BML-QDG-QA-047-FO4

7.0 TRAINING INVOLVEMENT

| Production and Quality Managers | QC Head |
| --- | --- |
| QA staff | Production Staff |

8.0 DISTRIBUTION AND CONTROL:

| MASTER COPIES | QUALITY ASSURANCE |
| --- | --- |
| CONTROLLED COPIES | ALL DEPARTMENT |

9.0 CHANGE HISTORY

| CURRENT REVISION | SUPERSEDE | REASON FOR REVIEW |
| --- | --- | --- |
| 00 | Nil | First issue |
| 06 | 05 | - Change in SOP format. - Inclusion of traceability of enclosures to parent SOP |