- OBJECTIVE:

To provide an instruction for retesting of raw materials and intermediates.

- SCOPE:

This procedure is applicable for retesting of raw materials and intermediates in QC department of Biomedical Ltd.

- RESPONSIBILITIES/ACCOUNTABILITIES:

QC Chemist, Microbiologist, Quality Control Head

- ABBREVIATION AND DEFINITIONS

RT – Retest

QC- Quality control

QA – Quality Assurance

LOD – Limit of Detection

- PROCEDURES

- After the receipt of re-testing intimation note from store department, QC Chemist takes its entry in the respective Retesting logbook and gives RT No. to raw materials or intermediate.

- RT No. Should be given as follows

For Raw material: BML/RM/RT/000/XX-YY

Where, 	BML 	= Biomedical Ltd

RM 	= Raw Material

RT 	= Re-test

- = R.T Number

XX	= year (e.g. 17-19)

- Collect samples as per sampling plan as per SOP BML/QDG/QA/060

- After completion of sampling, do the necessary critical tests (e.g. assay, LOD, water content, pH, specific optical rotation, impurities etc.) as per the Standard Analytical Procedure.

- Raw material may also be approved on the basis of physical appearance.

- After completion testing, the status of approved / rejected to be placed on the containers.

- Any raw materials shall be retested only twice during their shelf life.

- Frequency of retesting for raw material is every year.

- Retesting should be performing on first come first out basis. The retesting of material should carry out, whose due date for retest is earlier.

Materials, Equipment, Apparatus and Reagents

IPA (70%)

Raw material sampler

- REFERENCE AND ENCLOSURE

Enclosure 1-Log book for Re-test raw material- BML-QDG-QA-056-F01

7.0 TRAINING INVOLVEMENT

| All departmental managers | All sectional heads |
| --- | --- |
| QA Doc. Unit staff | All staff |

8.0 DISTRIBUTION AND CONTROL:

| MASTER COPIES | QUALITY ASSURANCE |
| --- | --- |
| CONTROLLED COPIES | ALL DEPARTMENT |

9.0 CHANGE HISTORY

| CURRENT REVISION | SUPERSEDE | REASON FOR REVIEW |
| --- | --- | --- |
| 00 | Nil | First issue |
| 06 | 05 | - Change in SOP format. - Inclusion of traceability of enclosures to parent SOP |