- OBJECTIVE:

To provide a standard operating procedure for Sampling of Packaging Materials (PM) in Biomedical Ltd

- SCOPE:

This SOP is applicable to the sampling of packaging materials.

- RESPONSIBILITIES/ACCOUNTABILITIES:

Store Officer, Quality Control Analyst, Quality Assurance Officers, Quality Control Head, Quality Assurance Manager

- ABBREVIATION AND DEFINITIONS

## NA- Not Applicable

## QA - Quality Assurance

## QC- Quality Control

## SOP - Standard Operating Procedure

## A.R. - Analytical Reference

- GRN- Goods Received Note

- LAF- Lamina Air Flow

- MSDS- Material Safety Data Sheet

- BOPP- Biaxially Oriented PolyPropylene

- AQL- Acceptable Quantity Limit

- PP- Poly-Propylene

- PVC- Poly Vinyl Chloride

- PROCEDURES

Gowning procedure for sampling of packaging material

- ﻿﻿Wear fresh gown before entering into sampling area.

- ﻿﻿Cover the hands with gloves and wear a cap and mask to cover hair and nose respectively.

- ﻿﻿Use separate apron, nose mask, Hand gloves, cap & shoe covers for the sampling of different material.

- ﻿﻿Mop the hands with 70% IPA and air dry before proceeding for sampling and particularly when samples are to be drawn for Microbiological testing.

Sampling

• On receipt of Packing Material in the quarantine through the receiving bay, the warehouse officer enters details of received material in the material management work-note. As soon as the Goods Inward Note (GRN) is prepared, and intimation is sent to the quality assurance analyst.

QA analyst shall record the details of the material into the AR log and generate the reference number for the material and inform the sampler with the intimation.

- ﻿﻿The sampler/Analyst will use packing inspection report sheet (Enclosure 2) will reach the warehouse department and request warehouse staff to take the material to be sampled at the sampling area.

- ﻿﻿The sampler/Analyst shall verify the cleanliness of the sampling area before bringing the material inside the sampling area and starting the sampling activity.

- ﻿﻿On arrival of Packing Material inside the sampling area, sampler will verify the material label, manufacturers' batch number, control number/A.R. No./ and ensure that material arrived in the room is the same material that is supposed to be sampled as per sampling worksheet.

- ﻿﻿The sampler/ Analyst will also ensure that the Packing Material is free from external damages. If there is any damage noted, it shall be marked separately and informed to the warehouse person, packing material section head, and Quality Assurance Person.

- ﻿﻿The sampler/Analyst will verify the Packing Material using a sampling worksheet to ensure the following:

- ﻿﻿Packing Materials shall have details of Material Name, Material Code, A.R. No., Material Manufacturer's name/ Vendor name, Manufacturer Batch No., Manufacturing Date, Expiry Date, Quantity of material received, Number of containers received, Pack Size, and Total number of Containers.

- ﻿﻿Materials shall not be accepted when the material name is not listed in the approved vendor list, if abnormality is observed during sampling, or Certificate of Analysis (COA) not received with the material.

- ﻿﻿The QA officer shall verify the COA received from the material manufacturer and compare it against the specification implemented in the organization. The QA officer will ensure that the analysis results of the material manufacturer are within the organization's specification limit. In case of any deviation noted, it shall be informed to the Quality Control laboratory head and Quality Assurance head to take a call on whether material shall be accepted or rejected.

- Before starting the sampling, ensure that the Sampling area environmental condition, such as Temperature, Relative Humidity, and Differential Air Pressure, is within the defined and acceptable

- Only one material shall be sampled at a time in Sampling Area.

- For sampling of primary packing material, ensure that Reverse Laminar Airflow (RLAF) is switched

"ON" at least 15 mins before sampling. Record the pressure differential across the pre-filter, intermediate, and HEPA filter and ensure that readings are within the defined range. In case of any abnormality, sampling shall not be started. Instead, inform to the engineering department to rectify before beginning the sampling activity.

- Containers to be sampled shall be taken within the safe zone of RLAF.

- ﻿﻿Before starting the sampling of material, the sampler shall follow safety precautions and shall once again inspect his gowning.

- ﻿﻿Material Safety Data Sheet [MSDS] shall be handy in the area as applicable.

- Before starting the sampling activity, make an entry in the logbook for sampling activity, such as material name, control number, LAF start time (in case of primary packing material), and sampling start

time.

- Reseal the container after sampling as under and transfer it. Draw the sample from different levels of the container. (i.e. From Top, Middle & bottom). Close the sampling point with butter paper and seal it properly using BOPP tape.

- Once the sampling activity is completed, enter the sampling end time. Entry or exit from the sampling area is restricted until the activity is completed.

- Entire sterile packing material sampled by the Microbiologist shall be sent to respective laboratories to carry out sterility testing and chemical testing, visual inspection check, physical tests.

- Keep the retain sample and sample to be analyzed at specified storage condition as per SOP.

- Mark ‘SAMPLED’ stamps on containers from which the samples are collected.

SAMPLED LABEL

Name of Material:_________________

Batch no.:_______________________

Mfg. Date:_______Exp. Date:_______

Supplied by:_____________________

Container no.:____________________

Sampled by:_____________________

- Affix yellow colour dully filled & signed ‘’UNDER TEST’’ label on each container below white label of the store.

- Prepare sampling record of material

- Make the entry of sampling shall be established with the formula below

| Sample Code | Total number of packs or containers received | Total number of packs or containers to be opened for visual inspection and collection of samples of analysis |
| --- | --- | --- |
| B | 02-08 | 3 |
| C | 09-15 | 5 |
| D | 16-25 | 8 |
| E | 26-50 | 13 |
| F | 51-90 | 20 |
| G | 90-150 | 32 |
| H | 151-280 | 50 |
| J | 281-500 | 80 |
| K | 501-1200 | 125 |

- Appropriate AQL for defects are classified as critical, major and minor.

- Critical defects are those which lead to adulteration or misbranding of the product and likely to cause product unfit for use.

- Major defects are those which can cause interference with machine operation.

- Minor defects are unlikely to any product damage or result in the product being unfit for use.

- If the packaging material dimension in out of limit found in minor variation from specification, send samples for machine trial.

- If the machine trial report found satisfactory, the material should be released

Retained samples

- Specimen of all the printed/unprinted packaging materials are retained 2 numbers along with respective analytical report

- PP, PVC, 25cm from each sampled rolls shall be retained along with respective analytical report.

- All the retained samples shall be kept up to 4-5years along with respective analytical report.

- REFERENCE AND ENCLOSURE

- WHO annex 8: Sampling of raw materials and packing materials.

- ISO 2859-1

- AQL 2.5

- Enclosure 1 – Packing Material Inspection checklist- BML.QDG.QA.061-F01

- Enclosure 2-List of Pack Materials sample quantity for analysis- BML.QDG.QA.061-F02

- Enclosure 3 – Packing Material Inward Register- BML.QDG.QA.061-F03

7.0 TRAINING INVOLVEMENT

| Quality Assurance Manager | Quality Control Head |
| --- | --- |
| QA Doc. Unit staff | Quality Control Analyst |

8.0 DISTRIBUTION AND CONTROL:

| MASTER COPIES | QUALITY ASSURANCE |
| --- | --- |
| CONTROLLED COPIES | ALL DEPARTMENT |

9.0 CHANGE HISTORY

| CURRENT REVISION | SUPERSEDE | REASON FOR REVIEW |
| --- | --- | --- |
| 00 | Nil | First issue |
| 06 | 05 | - Change in SOP format. - Inclusion of traceability of enclosures to parent SOP |