- OBJECTIVE:

To serve as a written guide in defining Quality Control activities at different stages.

- SCOPE:

This procedure is applicable all incoming and finished products at BIOMEDICAL LTD, Ilorin, Kwara State.

- RESPONSIBILITIES/ACCOUNTABILITIES:

Quality control Analysts, Quality Control Head, Quality Assurance Manager.

- ABBREVIATION AND DEFINITIONS

Nil

- PROCEDURES

- Organization has defined four stages of inspection named as stage 1, QC for incoming materials, stage 2, IPQC for in-process materials, stage 3 for Finished Goods inspection and stage 4 for post packing and pre-dispatch inspection.

- The Quality Control Department carries out the required inspection at various stages in accordance with the QAULITY PLANS.

Incoming material inspection

- The QC personnel receive the intimation for sampling and testing from the store in-charge along with supplier COA, if available.

- For incoming material, QC personnel enter the details in the incoming material Analytical Reference Book (LGB/QDG/CL/010/001-Infusion raw material, LGB/QDG/CL/010/002-Syrup raw material, LGB/QC/010/CL/003-Infusion Packaging material and LGB/QC/010/CL/004-Syrup Packaging material) for both infusion and syrup plants respectively) and assign QC A.R number to the received materials. The QC Analytical Reference Number shall be as given below;

RMF/AR/QC/YY/XXXXXX. OR RMS/AR/QC/YY/XXXXXX (Raw Materials).

Where;

RMF: Raw Material Infusion

RMS: Raw Material Syrup

AR: Analytical reference number

YY: Last two digits for that Year

QC: Quality Control

XXXXX: Serial Number and Material reference acronyms. See guide BML-QDG-CL-010-G01) for details.

PMS/AR/QC/YY/Material dedicated letter, serial number, volume or identification letter for the product (Infusion) OR PMS/AR/QC/YY/Material dedicated letter, serial number, volume or identification letter for the product (Syrup). See guide BML/QDG/CL/010/G03 for details.

Where:

- PMF: Packaging Material Infusion

- PMS: Packaging Material Syrup

- AR: Analytical reference number

- YY: Last two digits for that Year

- QC: Quality Control

- QC personnel shall take samples and conduct testing as per the standard requirement

- If the material meets all pre-defined specifications, then it shall be released for further action.

In-process Inspection

- During the in-process inspection, it is to ensured that line clearance must be performed as pre-defined procedure

- If there is any discrepancy in the output or the process, corrective actions must be implemented according to standard procedure and documented.

Finished Good Inspection

- For finished goods, QC personnel enter the details in the Finished goods Record Book (LGB/QDF/CL/019/001-Infusion, LGB/QDS/CL/019/001-Syrup) and assign QC Analytical Reference number to the finished goods. The QC report number shall be as given below;

BML/QA/FG/YY/Product code and BMR issuance serial number.

Where:

- BML: Biomedical Limited

- QA: Quality Assurance

- FG: Finished Good

- YY: Last two digits for the Year

- Product code:

- QC personnel shall take samples and conduct as per the standard testing procedure.

- If the product meets all pre-determined specifications, then it shall be released for further action.

Pre-dispatch Inspection

- During pre- dispatch inspection, QC must ensure that all the records related to the products are available and completed

- During pre-dispatch inspection, QA and store officer must ensure that the customer requirements are met and in compliance.

- REFERENCE AND ENCLOSURE

- SOP on Product Release: BML.QDG.QA.110

- Standard Testing Procedure: BML/QA/STP/FPF (INFUSION)

BML/QA/STP/FPS (SYRUP)

7.0 TRAINING INVOLVEMENT

| All departmental managers | All sectional heads |
| --- | --- |
| QA Doc. Unit staff | All staff |

8.0 DISTRIBUTION AND CONTROL:

| MASTER COPIES | QUALITY ASSURANCE |
| --- | --- |
| CONTROLLED COPIES | ALL DEPARTMENT |

9.0 CHANGE HISTORY

| CURRENT REVISION | SUPERSEDE | REASON FOR REVIEW |
| --- | --- | --- |
| 00 | Nil | First issue |
| 06 | 05 | - Change in SOP format. - Inclusion of traceability of enclosures to parent SOP |