RAW MATERIAL NAME: POTASSIUM CHLORIDE

SOP Ref. No.: BML-QDG-QA-105                                                            	 EFFECTIVE DATE: JAN., 2024

| S/NO: | TESTS REQUIRED | SPECIFICATIONS | SPECIFICATIONS REFERENCES |
| --- | --- | --- | --- |
|  | Description | White or almost white, crystalline powder or colourless crystals. | BP 2023 |
|  | Solubility | Freely soluble in water, practically insoluble in anhydrous ethanol. | BP 2023 |
|  | Identification | - It gives reaction of chlorides - It gives reaction of Potassium | BP 2023 |
|  | Acidity/Alkalinity | Not more than 0.5mL of 0.01 M hydrochloric acid or 0.01 M sodium hydroxide is required to change the colour of the indicator. | BP 2023 |
|  | Bromides | NMT 0.1% | BP 2023 |
|  | Sulphates | 300ppm | BP 2023 |
|  | Iodides | After 5 min, examine in daylight. The substance shows no blue colour. | BP 2023 |
|  | Aluminium | NMT 1.0 ppm | BP 2023 |
|  | Magnesium and alkaline-earth metals | Maximum 200ppm | BP 2023 |
|  | Sodium | NMT 0.1% | BP 2023 |
|  | Sulphate | ≤300ppm | BP 2023 |
|  | Loss on Drying | Maximum 1.0 per cent, determined on 1.000 g by drying in an oven at 105 °C for 2 h. | BP 2023 |
|  | Assay | 99.0 per cent to 101.0 per cent of KCl (dried substance). | BP 2023 |
|  | Total Microbial Count Total yeast &moulds Pathogens | ≤10CFU/ml ≤10CFU/ml Nil. Must be Absent | BP 2023 |
|  | Bacteria Endotoxin | ≤1.15OC (3 Rabbits sum response) ≤2.5EU/ml(LAL Test) | BP 2023 |

EU/ml = Endotoxin units per millilitre, ml = milliliter, LAL= Limulus Amoebacyte Lysate, NMT= Not More Than, CFU/ml= Colony Forming Unit per Millilitre